Notes for Memory Mapped Version 1.12 of VEDIT

These notes apply predominately to the SSM VB3 video board

and describe the patches and customization "Initialization

Bytes" needed to fully support the hardware attribute cursor

and bank select features.

Just specifying the "Initialization Bytes" is sufficient

to get the board up and running, however a very simple patch

will enable the VB3's attribute cursor.  Using the VB3 with

Bank Select requires a further patch.  This cannot be fully

described here, because it is very dependent upon other

hardware in the system.

When installing any of the patches, you should refer to

the enclosed file VEDITMEM.LST, which contains a listing of the

pertinent source code.

The VB3 needs to be initialized whenever the visual mode

is entered.  Specifically, the hardware scroll register must be

set to the last screen line in order to make the Base screen

address the upper left hand corner.  Since VEDIT produces its

own cursor on the screen, the default cursor must also be

disabled.  This is done by simply moving it to a column off the

screen.  The following sequence of instructions will perform

this initialization.  They may be specified as the

"Initialization Bytes" during customization, or may be

permanently patched into VEDIT at the label HCRSOF (see

listing).

OUT  0E0H    ;Enable video board. (Actually OUT KSTAT)

MVI  A,17H   ;Get # Rows -1. (17H is based on 24 line screen)

OUT  0D6H    ;Init scroll register. (Actually IOBASE + 6)

MVI  A,0FFH  ;Get value to put cursor off screen.

OUT  0DCH    ;Move default cursor off screen. (Actually IOBASE + 12)

OUT  0E1H    ;Disable video board. (Actually OUT KDATA)

RET

The equivalent Initialization Bytes would be:

Pair 1 :  00      E0

Pair 2 :  17      D6

Pair 3 :  FF      DC

Pair 4 :  00      E1

Instead of using the  default hardware cursor  on the VB3,

VEDIT creates   its   own   cursor.  The   VEDIT  documentation

describes three cursor  types:  0  =  blinking  underline,  1 =

blinking reverse video block and 2 = solid reverse video block.

Of these, only type  0 will work  with the VB3.  However, VEDIT

has one additional cursor type especially for the VB3.  This is

the hardware attribute cursor,  or cursor type  "3", and may be

selected as the default  cursor during customization  or may be

changed with  the  "EP"  command.  To  implement  this hardware

attribute cursor, you  will  need  to  patch  one  byte  in the

VEDITZM.SET  or  VEDIT8M.SET   file  before   you  perform  the

customization, or else in the  VEDIT.COM file after you perform

the customization.  The byte to be patched is in the line after

Page 2

CURSO4 in the listing.  The listing  shows the instruction "DAD

B" with the  object  byte  of  "09".  This  "09"  byte has been

changed to "00"  in your ".SET"  files and needs  to be changed

back to "09"  to implement  the attribute  cursor.  (Our reason

for disabling cursor type 3 is  to prevent problems for non-VB3

users.  Otherwise, should a  non-VB3  user  accidentally select

cursor type 3,  VEDIT  would  begin  to  "mysteriously"  eat up

programs in  memory,  possibly  itself,  but  very  likely  the

operating system.)

The following steps show  how  to  use  DDT  to enable the

attribute cursor in the VEDITZM.SET and VEDIT8M.SET files:

B>DDT VEDITZM.SET

DDT VERS. 2.0

NEXT  PC

2900 0100

-S1DF2

1DF2  00 09            ;DDT prompts with 00, you type 09 and <RETURN>

1DF3  77 .             ;You type "." and <RETURN>

-<CTRL-C>

B>save 40 VEDITZM.SET

B>DDT VEDIT8M.SET

DDT VERS. 2.0

NEXT  PC

2B00 0100

-S1FD7

1FD7  00 09            ;DDT prompts with 00, you type 09 and <RETURN>

1FD8  77 .             ;You type "." and <RETURN>

-<CTRL-C>

B>save 42 VEDIT8M.SET

To implement  screen  bank  select,  you  must  patch  the

necessary code in the space provided  at the routines SCRIN and

SCROUT.  In SCRIN, the code will need to include "OUT KSTAT" to

enable the VB3, and  any necessary  code to  disable the memory

overlapping the  board.  In  SCROUT,  the  code  will  need  to

include "OUT KDATA" to disable the  VB3, and any necessary code

to  enable  the  memory  overlapping  the  board.  If  the  VB3

overlaps the main  text buffer  of VEDIT,  which is  all memory

between the end  of VEDIT and  the beginning of  the CP/M BDOS,

you must also change the byte at BNKSEL from a "00" to a "01".

Please contact us  should you require  any assistance with

these patches or  with  any  other  aspect  of  VEDIT. We would

appreciate any  comments  you  might  have  regarding  the bank

select and hardware  attribute cursor,  since we  would like to

make this part  of  the  customization  as  soon  as  we find a

technique which is general  enough to work  with most available

video boards.

#############################################################################################################################