Notes for Polytechnic VTI display board

The POLY VTI display board requires the  high order bit to be set

for all  normal displayable  characters.  In order  for VEDIT  to work

properly with this board,  the applicable patch  below (depending upon

Z80 or 8080  version) must be  made to the  VEDITZM.SET or VEDIT8M.SET

file before the customization is performed.  During customization, the

value of "AD" should also be specified for the continuation character.

B>DDT VEDITZM.SET

DDT VERS. 2.0

NEXT  PC

2900 0100

-S1C26

1C26  00 F6

1C27  00 80

1C28  77 .

-S2569

2569  20 A0

256A  2D AD

256B  20 A0

256C  B1 .

-[CTRL-C]

B>SAVE 40 VEDITZM.SET

B>DDT VEDIT8M.SET

DDT VERS. 2.0

NEXT  PC

2B00 0100

-S1DE8

1DE8  00 F6

1DE9  00 80

1DEA  77 .

-S2789

2789  20 A0

278A  2D AD

278B  20 A0

278C  B1 .

-[CTRL-C]

B>SAVE 42 VEDIT8M.SET

##############################################################################