FROM WARD CHRISTENSEN:

HERE IS HOW TO MODIFY THE CPM ED.COM PAGE (P) COMMAND

FOR OTHER THAN 24 LINE DISPLAYS.

FOR EXAMPLE, IF YOU HAVE A VDM, YOU WOULD LIKE 15 LINES, NOT

23, TO BE SHOWN.

DDT ED.COM

DDT VERSION 1.3

NEXT   PC

1500  0100

-LC95 C96

0C95 MVI M,17

0C97

-AC95

0C95 MVI M,F

0C97

-^C

A>SAVE 20 EDP.COM

THAT'S ALL THERE IS TO IT.

####################