User's manual for CLINK Version 3.0

CLINK is a CP/M transient program which allows the owner of a CP/M

system with a D.C. Hayes 80-103A modem to establish a communications

link to a wide variety of remote devices, such as:

1) commercial timesharing services

2) university computing centers

3) "community bulliten board" systems

4) "remote console" hobbyist systems

5) other CP/M users runnning CLINK

The only restrictions on type of remote devices are:

1) must be 110 or 300 baud asynchronous

2) must be Bell 103A compatible

3) must be "teletype" environment

All other characteristics which may vary from site to site are user

programmable in a very simple manner.

Bringing up CLINK in your own CP/M system is as simple as making a

copy on one of your own diskettes and running it. All console and disk

I/O is done via BIOS and BDOS calls. All modem I/O is done via the

"standard" 80-103A addresses (80H,81H,82H). This device independence

has caused one restriction: the (effective) baud rate of the console

device must be much greater (4x or more) than the selected baud rate

of the modem. This is normally no problem, but simple changes can be

made (which make CLINK device-dependent) to allow use of a console

which runs at the same baud rate as the modem. This requires a good

working knowledge of 8080 assembly language.

Some background information on data communications is necessary

before proceeeding to the detailed description of the available

directives. Several good books are available which cover the subject

very well, starting at a level that the typical hobbyist should be able

to follow with little or no difficulty. Two of the best are:

1) Signetics Data Communications Handbook ($3.50)

Signetics Corp. / P.O. Box 9052 / Sunnyvale Ca 94086

2) Technical Aspects of Data Communication ($17.95)

John McNamara / Digital Equipment Corporation

The following makes no attempts to cover all the information in the

above volumes, but should be sufficient to allow intelligent use of

most of the options available in CLINK.

The "Data Communications" field has evolved from simple beginnings

in the days of teletype loops into a fairly complex set of conventions

and standards for allowing various devices (terminals, modems, computers,

the phone system, etc) to be hooked together so that data may be

transferred from one point to another. Like any complex technical field

data communications has its own unique terminology. Some of these terms

will be defined in this manual.

Parallel Digital Data - simultaneous transmission of an entire byte (or

machine word) using two-state voltage levels, usually in conjunction

with "handshaking" signals, such as "data available", etc. The link

between the S100 bus and the UART on the 80-103A is "parallel".

Serial Digital Data - time-multiplexed data on a single line, such that

the various bits that make up a byte are placed on the line, one after

another, until all bits have had their "time-slice". The link between

the UART and the modem on the 80-103A is "serial".

Asynchronous Serial - one set of conventions for time-multiplexing

data. Allows the data to occur at sporadic intervals, up to some

maximum frequency. Basically, this allows an arbitrarily long interval

to elapse from the end of one character to the start of the next.

Typical of situations where a human is involved (who can not be expected

to continuously type, or read, a fixed number of characters per second).

Requires overhead information on each character to indicate "start of

character" and "return to idle line". Not very demanding of timing,

typically within 5%. Used in relativly low speed environments (75 to

9600 baud). The link between the UART and the modem on the 80-103A is

digital asynchronous serial.

Synchronous Serial - another set of conventions for time multiplexing

data. Requires the data to occur at fixed intervals (determined by bit

rate). Basically as soon as one character has ended, the next must

immediately begin (no gap between). Typical of machine to machine links.

Requires timing information (clocks) alongside serial data. No overhead

is required on each byte, just one or more "synchronization" bytes at

the start of a block of data to establish where the "byte" boundaries

are within the continuous bit stream. Typically used in relatively high

speed environments (1200 to 56K baud). Very little hardware (or software

for that matter) is currently available on the hobbyist market which can

support this technique (exception: IMSAI SIO2 I/O board).

UART - Universal Asynchronous Receiver / Transmitter. Single LSI chip

capable of simultaneously serializing (parallel to serial) and/or

deserializing (serial to parallel) asynchronous serial data. One of the

basic building blocks of many communications systems. Ususally allows

selection of details of the conversion process, such as number of data

bits, number of stop bits, etc. The 80-103A has a fairly typical UART,

and provisions for controlling the various options in software, by

outputting 1's or 0's to particular bits in control registers.

Data bits - the number of non-overhead bits in each asynchronous

character. Typically seven for most commercial timesharing services.

For special purposes, such as using 8-bit codes like EBCDIC, or

transferring 8-bit data (such as object code, compressed text, etc)

this may be extended to eight.

Stop bits - the number of bit-times that the line is guaranteed to

remain in the "mark" state (logic 1) after a given asynchronous

character. For 110 baud, this is usually two. For all other conditions,

one is almost always sufficient and yields higher throughput.

Parity bit - an error detection mechanism used in most asynchronous

communication systems. Basically, one additional bit is appended

after the data bits, but before the stop bit(s). If using "even" parity,

this bit will be chosen so that the total number of "on" (logic 1) bits

in the data+parity part of the character, is an even number. If using

"odd" parity, this bit will be chosen so that the number of "on" bits

is an odd number. When the receiving end looks at a character which has

been transmitted with parity, if a single (or in fact 3,5,7) bits have

changed state due to line noise, the "parity" of the received character

will be wrong, and the device (or program) can act accordingly.

Unfortunately, if 2 bits (or 4,6 or 8) change state, this technique

fails to detect anything amiss. Fortunately, most line glitches only

affect a single bit.

Modem - MOdulator / DEModulator. Hybrid digital/analog circuit used to

transduce "voltage level" serial digital data into (from) voice-range

audio tone, which may be sent over the commercial phone network without

undue distortion or disruption of other phone company services. Ususally

uses a modulation technique called "frequency shift keying" (FSK) in

which a "carrier" frequency, in the 1 to 3 kHz range, is shifted up or

down about 500 Hz by logic 1 or 0 respectively. A Bell 103 compatible

modem uses this technique, and in fact, typically uses two carrier

frequencies which may be simultaneously modulated to support full duplex

(bidirectional communication). To be 103 compatible, particular carrier

frequencies and shifts must be employed as specified by Bell. Most 110

and/or 300 baud asynchronous modems, including the modem circuit on the

80-103A are Bell 103 compatible.

Originate Modem - one end of a 103 modem pair. Typically the end which is

associated with a remote terminal. Modulates (transmits on) the lower

carrier frequency, demodulates (receives on) the higher frequency.

Answer Modem - the other end of a 103 modem pair. Typically the end

which is associated with a central computer. Modulates (transmits on)

the higher carrier frequency, demodulates (receives on) the lower

carrier frequency.

Half Duplex - sending information over a communications line in only

one direction at a time. Requires having a terminal which is connected

to such a line locally echo any typed characters to the terminal's

display unit. In a CLINK to CLINK connection, the answer end is forced

to half-duplex.

Full Duplex - sending information over a communications line in both

directions simultaneously. Allows having the "host" system echo any

characters typed on the remote terminal. This assures higher reliability

as the characters you see on the display unit of the terminal have made

it all the way through the communications line and back. The probability

of a character changing to something else (because of noise, lost bits,

etc.) being received incorrectly by the host system, and accidently

changing back to the original character on the way back is effectively

zero. This facility may also be used, by sophisticated software, to send

two different files at once, one from host to remote, the other from

remote to host, thereby saving on long distance charges. Running full

duplex also allows the host system to selectively not echo certain

characters (such as passwords) or substitute a different character(s)

for any given character received from the remote terminal. The 80-103A

will support either full or half duplex.

Auto-Dial - allowing a computer (or intelligent terminal) to generate

the clicks or touch tones which the switched telephone network uses to

establish links between any two phones. The 80-103A has facilities for

dialing by generating carefully timed clicks (on-hook / off-hook).

Auto-Answer - allowing a computer (or intelligent terminal) to answer

the phone in response to an incoming call. This is implemented by

sensing the ring voltage (about 70 VDC) on the phone line and raising

a flag (status bit) which the computer can sense (or be interrupted by)

along with a relay which the computer can open to go "off-hook". The

80-103A has this facility.

Protocol - set of rules for message format and handshaking imposed on

the transmission of data to and/or from one device to another, for the

purpose of detecting and correcting errors, and allowing controlled

start/stop of the data without loss of characters. There are quite a

few protocols currently in use in the computing industry, and even a

few (mostly proposed, rather than implemented) in the hobby field.

Checksum - simple error detection device used by some protocols, usually

just the sum of the bytes in a message, modulo 256. Sometimes referred

to as longitudinal parity, or longitudinal reduncancy check (LRC).

Suitable for detecting all 1 bit errors, and 97% of all possible errors.

CRC - Cyclic Redundancy Check. More elaborate error detection scheme,

which is capable of detecting 99.9987% of all possible errors. Used in

many industrial protocols, including IBM bi-sync and SDLC. Several

subtle variations are currently in use, the only difference being the

'polynomial' used in the algorithm. The most common by far is CRC-16,

followed by CRC-12. In fact, an LRC is merely a degenerate CRC, and

can be generated by the same algorithm, with a very simple polynomial.

Normal Use of CLINK Version 3.0 (Originate)

If you have already created a configuration file (with ED or with

the "SAVE" directive in an earlier CLINK run), you can specify any or

all options at once by specifiying this file on the command line used

to start CLINK, as the first parameter. If such a file is specified,

CLINK will read the lines one by one, echo them, and process the

directives on them. If an error is detected in any line, CLINK will

notify you after that line is echoed, then abort. If created with a

text editor, this file consists of one or more lines with one directive

per line. Each directive consists of one keyword followed by a delimiter

(one or more spaces and/or tabs) followed by a legal parameter for that

keyword.

Once the file (if any) has been read, you will be asked to enter any

new (or specific) directives. When all such directives have been entered,

type CR (carriage return) immediately after the prompt (empty line). At

this point, if a carrier is already present, the link is immediately

resumed.

Normally, at this point, if a number was specified, it will be dialed.

If a carrier is detected within 30 seconds, the link will be initiated,

else you will be notified (NO CARRIER DETECTED), and CLINK will abort.

Once the link is established, characters typed on the console are

written to the modem, and characters read from the modem are written to

the console.

If an "ESCAPE" character was defined before the link was established,

then anytime that character is typed on the console, CLINK will

temporarily escape from the link (carrier remains on) to allow you to

enter a "local command". This can be any of the configuration directives

or one of several additional commands which are useful only once the

link has been estalished. The recommended value for ESCAPE is 1BH. This

value should be chosen so as to not conflict with any of the characters

needed by the remote system.

If a "REMESC" character is defined at any given time, then at any

time after that, when that character is received from the modem

(presumably typed by the remote user), CLINK will allow the remote

user to enter a 'local' command which affects your CLINK. When

you dial into another CLINK system (which is running in ANSWER mode)

it will tell you if a REMESC character is defined, and if so, what

hex value it is.

If a "BREAK" character is defined at anytime, thereafter anytime

that character is typed, the communications line will be broken for 150

milliseconds. This function is useful on certain time-sharing systems,

especially those of somewhat earlier vintage (to be polite).

If an "ERROR" character is defined at any time, thereafter, that

character will be substituted for any character read from the modem

with any type of error (parity, overrun or framing). At all times,

counts are maintained of each of these errors. These may be viewed at

any time with the 'ERROR S' command, or cleared to zero with 'RESET'.

Local Commands

The following commands are available if the ESCAPE character has

been defined during initialization. To issue a local command, merely

type the character specified as the 'ESCAPE' code on the local console.

A prompt  ->  will be displayed, in response to which the local command

may be typed, terminated by a carriage return. Once the local command

is accepted and processed, the communications link is resumed. Note

that at no point (except with the ABORT command) is carrier dropped.

If a REMESC character is defined, then anytime that character is

received from the modem a prompt will be displayed on the console of

the remote system, at which point the remote user can enter a 'local'

command which affects your copy of CLINK.

RETURN		Return to CP/M without dropping carrier. The link may

be resumed after local processing by just executing

CLINK again with the same configuration file. CLINK

will detect the presence of the carrier and jump into

the main loop of the link without re-dialing.

ABORT		Drop the carrier, hang up the phone, and exit to CP/M.

ERROR SUMMARY	Request a summary of the error counts (Parity, Overrun,

and Framing) as detected since start of link (or last

RESET command). Note that only the first character of

SUMMARY is checked for.

RESET		Reset error counts to zero.

DEBUG ON	Enable hex debugging mode. All incoming characters are

displayed in hexadecimal on the console, 16 per line,

rather than in ASCII. This allows any non-printing

characters (control codes, pad characters, etc) to be

easily identified. Hex to ASCII tables are readily

available, hence will not be reproduced here.

DEBUG OFF	Disable hex debugging mode, if enabled.

LIST		List current configuration settings to Console.

SAVE D:FN.FT	Write current configuration settings to specified

CP/M file, for later use as a setup file for CLINK.

DIR		Same as CP/M DIR command, including ambiguous file

names (? and * wildcards), but used from within the

CLINK program. Prints file names 4 per line.

CAPTURE		Start capturing incoming characters into memory, and

keep track of number of characters captured. Will not

allow BDOS to be overwritten.

WRITE D:FN.FT	Write captured text, if any, from memory to specified

CP/M file.

READ D:FN.FT	Read specified CP/M file and send it as if it were

coming from the local console keyboard. Anytime a

carriage return is sent, CLINK will await return of

a line feed, and optionally ignore n additional

'garbage' characters, then continue with the next line.

If in FULL DUPLEX, each character is checked against

the original as it is echoed by the remote system.

At end of file, an error summary is printed and normal

link operation is resumed. A READ operation may be

aborted at any time by typing any character on the

console of the 'sending' system.

GARBAGE nn	Set 'garbage' count to nn (hex). See READ command.

EXPAND ON	Select option whereby tabs are expanded to an

appropriate number of spaces as they are transmitted

via the READ command. Useful for transmission of

files with embedded tabs to non-CP/M systems.

EXPAND OFF	De-select tab expansion option (default state). This

will greatly speed up transmission of text with tabs

to other CP/M systems, without loss of information.

SEND D:FN.FT	Send specified file using the MYCROFT protocol to

another system running the RECV command in CLINK.

May be used from either ANSWER or ORIGINATE system.

May be used to send any type of file (including .COM

or other 8 bit data). Will force 8 data bits, if not

already set. Will detect virtually any possible error,

and automatically retransmit when necessary, informing

the user when it has done so. Will also timeout when

an expected response is not detected in a reasonable

length of time, and attempt to recover. May be aborted

at any time by typing any character on the console.

Will inform user when file has been sucessfully sent.

RECV D:FN.FT	Receive file being transmitted with MYCROFT protocol.

May be used from either ORIGINATE or ANSWER system.

Will force 8 data bits if not already set. Will inform

user when any error is detected, and automatically

request other end to re-transmit. Will inform user

when entire file has been received intact.

BLOCK nn	Set block size to nn (hex) 256 byte pages for SEND

command. Default is 4 pages (for 1024 byte blocks).

Given an error free line, the larger this value is,

the quicker the file will be sent (values over 8 are

not recommended for general use). On a noisy line,

the smaller this value is, the less will have to be

re-transmitted when an error is detected.

Normal Use of CLINK Version 3.0 (Answer)

If CLINK is brought up in ANSWER mode, it will not dial out even if

a number was provided, but rather will await an incoming call. Once

someone dials in, CLINK will respond with an ANSWER carrier. If, within

30 seconds, an ORIGINATE carrier is detected, a normal communications

link is established, and the ANSWER system will resond with a greeting

which includes the CLINK version number, and what the REMESC character

is, if there is any.

Note that the ANSWER system must run in HALF DUPLEX, as the terminal

or ORIGINATE system will not echo characters sent by the ANSWER system.

File transmission is somewhat simpler from ORIGINATE to ANSWER, but it

will work in either direction. If a REMESC character is defined, an

unattended CLINK ANSWER system can be used as a central repository and

distribution point for text files (see DIR, READ, CAPTURE and WRITE).

Note that remote 'local' commands work only from an ORIGINATE to an

ANSWER system, not vica versa. This is due to conflicts with HALF vs.

FULL DUPLEX and who echos whose characters when. Otherwise, ANSWER mode

operation is identical to ORIGINATE mode operation. It is recommended

that a single configuration file 'ANSWER' be setup on the CLINK disk

so that an ANSWER system can be quickly brought up by the command:

CLINK ANSWER

Note also that no auto baud detect is available yet in ANSWER mode.

File Transmission Using MYCROFT Protocol

To transmit a file from one CLINK system to another, one may simply

establish a normal link (as described above), then have the receiving

system enter the local command 'RECV D:FN.FT' (where D:FN.FT is the

CP/M file name of the file to be written), and the transmitting system

enter the local command 'SEND D:FN.FT' (where D:FN.FT is the name of an

existing file on the transmitting system's disk). From that point until

CLINK informs both users that the transmission is complete, no further

interaction is required on the part of the user, unless the sender

decides to abort the transmission before the entire file is transmitted.

Informative messages may be printed as possible errors are detected and

corrective action is taken by both systems, but no response is required

from either user. If a large number of retransmissions are required, it

is suggested that the block size be reduced with the BLOCK command.

Description of MYCROFT Protocol

CLINK uses a simple asynchronous protocol to transmit files with

reliable error detection and automatic retransmission of bad data. It

is very loosely based on DEC's DDCMP. There are four possible messages

which may be used at various times during a file transmission:

Data Message - contains typically 1024 bytes of data

Acknowlege - used to acknowlege reception of an intact message

Negative Acknowledge - used to request re-transmission

End of Transmission - used to indicate end-of-file

All messages start with an SOH (ASCII Start Of Header). The second byte

is a message type code (NUL, ACK, NAK or EOT). On all messages but the

data message, a two byte CRC then follows, high order byte first. On

data messages, after the NUL code, a two byte message length (in bytes,

high order byte first) follows, then an STX (ASCII Start of Text), then

the actual data bytes, then an ETX (ASCII End of Text), then the two

byte CRC, as before. In summary, the messages are as follows:

SOH NUL hicnt locnt STX data data ... data ETX CRC CRC

SOH ACK CRC CRC

SOH NAK CRC CRC

SOH EOT CRC CRC

Note that the range of the CRC (that is, the data which is used in

building it) encompasses all bytes in a message except the SOH and the

CRC bytes themselves.

Sample File Transmission (on a VERY noisy line)

Msg 1	--->	ok		normal reception

ok	<---	ACK		normal acknowledge

Msg 2	--->	error		error detected

ok	<---	NAK		retransmit request

Msg 2	--->	ok		retransmission

error	<---	ACK		ACK received in error

NAK	--->	ok		retransmit request

ok	<---	ACK		previous msg re-sent

Msg 3	--->			message lost

Msg 3	--->	ok		resent after 10 sec timeout

ok	<---	ACK		normal acknowledge

Msg 4	--->	error		error detected

<---	NAK		response lost

ok	<---	NAK		resent after 10 sec. timeout

Msg 4	--->	ok		normal re-transmit

ok	<---	ACK		normal acknowledge

EOT	--->	ok		That's all folks

ok	<---	EOT		EOT acknowledge

79/01/14. LEH.

###########################