DU-V74.DOC

by Ward Christensen

with additional notes by Ron Fowler

(revised 1/15/81)

HOW TO USE DU ver 7.4 DISK UTILITY

INSTALLATION:

The 7.0 and later versions of DU are designed to

be installed with a minimum of trouble.  In fact, in al-

most all cases, no changes to the source file should be

necessary to get DU up and running. This is because DU

uses the disk paramter block of CP/M to determine the

characteristics of the disk environment.  (see the dis-

claimer for certain non-standard versions of CP/M 1.4).

The only parameter that should need to be changed

will be the clock speed flag at 103H.  Leave this byte

zero if you have a 2 mhz clock.  Patch it non-zero for

4 mhz.  This is only needed for the "Z" (sleep) command.

USE:

Any valid command string may be placed as an

operand of the original DU command, i.e.:

A>DU G0;D;G2;=OK<D><A><1A>;D

Functions supported:

Tnn	Seek to track nn (no read)

Snn	Position to sector nn, and read

Gnn	Position to group nn and read.

G	Shows current position

V	Views the current sector.

(assumes ASCII data)

Vnn	Views nn sectors

Fname	print directory for file "name",

then positions to it's directory

sector. (Won't position under

CP/M 2.x, but see = command below.)

=string	Ascii search, starting at current

sector. <xx> hex may be imbedded,

or used alone:  To find "IN 0FEH":

=<db><fe>

Ignores bit 7 unless <xx> is used.

L	Re-logs in the current disk. You may pull

out a disk, put in a new, and "L" just

to log it in.

Lx	Logs in disk 'x', such as: LB

Ux	Logs user 'x' for next F command.  Gives

'?' error if not CP/M version 2.x.

D	Dump sector, hex + ASCII

A	Dump sector, ASCII only

H	Dump sector, hex only

note all dump commands (D, A, H) may be optionally

followed by a starting and ending address:

D0,7F	is the same as just D

D3,5

A20,3F

CHaddr,val,val,val... change hex in sector

CAaddr,char string... change ASCII in sector

NOTE that <xx> may be hex imbedded

in the Ascii:  ca0,OK<d><a><1a>

----> Use W to write changes to disk.

Note that the C command echoes

the overlaid data for verification.

CHaddr-addr,byte

or	CAaddr-addr,byte	repeats a change

+	advance 1 sector (if below track 2,

this advances to next numerical, if

2 or >, advances based on CP/M's normal

sector scrambling algorithm, i.e. so +

will get the next logical sector of the file

-	backs up 1 logical sector

Note + and - may take an amount:

for example, +F steps in 15 sectors.

Note also that "-" issued at the first logical

sector of the disk will wrap back to the last.

Further, "+" issued at the last sector will

wrap forward to the first.

?	Gives command summary

#	Prints the disk parameters

M	Dumps a map of the group allocations

for files.

Mn	Shows which file is allocated to

group "n".

N	Resets CP/M via the BDOS.  This may

make it possible under some implementations

of CP/M to change the disk format (e.g., density,

sides, etc)

R	Reads the sector currently positioned to

into memory.  Note R (Read) is implicit in

the G, +, and - commands, but N-O-T in the

S and T commands (I did it because I was

tired of disk reading after T command before

I had a chance to issue the S command)

W	Write back the current sector (N-O-T-E may

not be used after an F command, as CP/M was

used to find the file in the directory

X	Exit back to CP/M (Must press return).  Ctl-c

was too easy to hit over modem lines, so I

decided on 2-byte (X, CR) to exit.

P	Toggle printer switch on/off

Z	Sleep - causes the program to pause, such

as to look at a dump.  Z is 1 sec.  Znn

is nn tenths of a second on a 2 MHz 8080.

<	Saves current sector in a save buffer

>	Gets saved buffer.  < and > may be used

to move a sector to another place.

/	Repeats entire command.  Defaults

or	/nn	to "forever".  NN may be 2 to 65535

multiple commands may be separated by ";"

Example: the following commands will erase the

b disk directory to all E5's:

lb		log in b drive

g0		position to dir.

ch0-7f,e5	fill with e5

<		save the sector

>;w;+;/16	restore, write, next,

repeat 16

----This could be shortened to:

lb;g0;ch0-7f,e5;<

>;w;+;/16

END

##############################################################################