ZX65: Simulating a Micro					Page 1

Having logged many hours of software development work on my Z80-

based system, I recently decided that I was up to a particular challenge:

using my system as a development tool for a totally different microprocessor;

specifically, the 6502.  This decision, I must confess, was not altogether

based on a burning lust for knowledge, but rather on a more practical re-

quirement:  I had become involved in the development of a 6502-based device,

and had no ready access to a 6502 development system.  I decided that a sim-

ple cross-assembler wasn't good enough, and set out to write a 6502-to-Z80

object code translator/simulator/interpreter.

The program described here, which I have dubbed ZX65, is the result

of my efforts to generate a software package to perform this task.  It is

useful not only for the intended purpose of software development, but also

for the rewarding experience of exploring a whole new field of good soft-

ware written for a processor other than one's own.

ZX65 operates as an interpretive simulator; that is, a simulated

6502 CPU is maintained in system memory, and each encountered 6502 instru-

ction is decoded and processed interpreter-fashion to properly act upon the

simulated CPU registers.  The entire package resides in just over 4K Bytes

of high system memory, leaving the remaining portion, including the all-im-

portant (for 6502) base page, free for program and data storage.

ZX65: Simulating a Micro					Page 2

Note that CP/M* serves only as a loader and is not used thereafter.  In fact,

while the program loads normally into CP/M's Transient Program Area, it imm-

ediately relocates itself into high memory, overlaying both the CCP and the

BDOS portions of CP/M.  The user I/O linkages (CBIOS), however, remain intact

and are used by the package to perform disk and console functions.  The func-

tions which must be provided by the system are a subset of those required by

CP/M and should thus be readily available.  These functions are all accessed

via a jump table within the interpreter and thus can be readily changed to

accomodate different system requirements (see table 1).  Parameters must be

passed to the drivers in register "C" (register pair "BC" for item six) and

data, if any, is returned in the accumulator.

ZX65 is referred to as a package since it includes three distinct

function groups.  The first and largest is the interpreter/simulator itself.

The second group is an elementary monitor providing the functions of memory

examine and modify.  The third group consists of a self contained, elementary

disk operating system (DOS). (Note: ZX65 files are not CP/M compatible.) The

normal configuration assumes a two-drive system, with the system diskette on

drive A, and all internal disk access routed to drive B.

ZX65: Simulating a Micro					Page 3

(Modification for a single drive system could easily be done, requiring the

user to switch diskettes after the package is loaded.)  The package is writ-

ten in standard Zilog/Mostek source code and is quite heavily commented.  Much

use is made of the Z80's special instructions and index registers, and oper-

ation on an 8080 is definitely not possible without extensive modification.

Although the commented listing should guide you through the program's opera-

tion without too much trouble, a brief description of the interpreter portion

is in order. A study of the small but powerful instruction set of the 6502

will reveal certain patterns in the op codes, as also occurs with most other

processors. The eight "BRANCH ON CONDITION" codes, for example, all have zero

as a low-order nybble, while the high-order portion is always an odd value.

Another pattern, not so obvious, is that every instruction having an odd-

valued op code uses multiple addressing modes.  These and other such patterns

are exploited by the program.  Even so, much of the code is devoted solely to

decoding the op code and transferring control to the proper instruction proc-

essor.  The code beginning at "STEP" (line 241) performs the initial decoding,

and fully decodes certain instructions such as "RTI", "JMP", and a few others.

In most cases, decoding proceeds to the point where the instruction is known

to be part of a group, such as the branch group described above.  In these

cases, look-up tables are used, but not always in the same way.

ZX65: Simulating a Micro					Page 4

The branch group and the flag set/reset group, for example, derive mask

patterns from the tables which are used for testing, setting, or resetting

the condition flags.  In the case of multiple addressing mode instructions,

however, the tables serve an entirely different purpose.  The table contents

are, in these instances, decoded into addresses for indirect jumps; first to

the addressing mode processors, and then to the actual instruction processors.

Each processor performs its instruction as required.  The "ADC" instruction,

as an example, causes a byte of data to be fetched from memory or from the

object code, depending on the addressing mode.  After setting the Z80 carry

flag to match the simulated 6502 carry flag, the data byte is added to the

value currently in the simulated accumulator, and the result is stored there.

The simulated carry flag is then updated to again match the Z80 carry, and the

simulated sign, overflow, and zero flags are set or reset according to the re-

sults in the accumulator.  (In this case, the overflow flag is set or reset

according to a logical "OR" of bits six and seven.)  Considerable care has

been given to treatment of the flags, since Z80 flags and their 6502 name-

sakes do not always have the same meaning.  (Case in point:  the 6502 carry

flag assumes a logical "NOT" borrow meaning during subtract and compare op-

erations, while the Z80 and most other common processors invert the carry

flag internally during these operations so that it represents a "TRUE" bor-

row. Other flag differences, for the most part, are more subtle.)

ZX65: Simulating a Micro					Page 5

Finally each individual processor is responsible for exiting with codes

for the mnemonic, the addressing mode, and the number of bytes occupied by

the instruction available for use by the "DISPLAY" routine (line 173) to in-

dex into yet another set of tables and retrieve the ASCII messages for mnem-

onic and addressing modes.  Upon exiting the instruction processors, the val-

ue stored in "NEXT PROGRAM COUNTER" is updated, and control is passed back to

the COMMAND section where the operating mode in effect determines whether to

display the updated CPU contents, and whether or not to continue execution

with the next instruction.

Several distinct operating modes are available, defined briefly as

follows:

SINGLE-STEP MODE:  One instruction is decoded and executed. The console

then displays the address of the instruction just executed (Current

Program Counter), the decoded instruction (Mnemonic and Operand),

the state of all CPU registers, and the address of the next instr-

uction to be executed (Next Program Counter).  Control is then

returned to the user.

MULTI-STEP TRACE MODE:  The user specifies up to 255 instructions to be

executed with the results displayed as above following each step.

This occurs at the rate of approximately two instructions per sec-

ond.  Control is returned to the user after the required number

of steps have been executed and displayed.

RUN-TO-BREAKPOINT MODE:  Instructions are executed without display up to

and including the user-specified breakpoint.  CPU registers are

then displayed as above, and control is returned to the user.

ZX65: Simulating a Micro					Page 7

FREE-RUN MODE:  Instructions are executed with no display and no break-

points.  The console keyboard, however, is monitored, and CPU con-

tents are displayed as above and control is returned to the user

at any point upon console input.

The user may examine any or all CPU registers, including the Pro-

gram Counter, at any time that he has control, without otherwise disturbing

any operation on progress.

The 6502 "BRK" instruction (software interrupt) is handled through

a vector location, just as in a normal system.  Optionally, however, the BRK

instruction may be used (and is initialized) to return control to the user

in the same manner as the breakpoint mode described above.  The BRK instru-

ction and breakpoint mode operate independently, and both may be used in the

same program.  Additionally, up to five different "user" subroutines may be

called for transfer of control to Z80 routines (such as console I/O).  These

are invoked by executing either a "JMP" or "JSR" instruction to addresses

0000, 0001, 0002, 0003, and 0004 (HEX).  Control is passed via a jump table

located within the interpreter, which the user may load with Z80 "JP" ins-

tructions to the desired subroutines.  The subroutines must be terminated

in the normal Z80 fashion by one of the "RET" instructions.  Note that the

first two entries in the table (JMP or JSR 0000, 0001 HEX) are initialized

as console input and output via the 6502 accumulator.  Three parameters may

be passed to and from all user subroutines, as outlined in table 2.  Table

3 lists the locations of the BRK vector and the user subroutine jump table.

ZX65: Simulating a Micro					Page 8

Although the interpreter is quite versatile and all 6502 instruc-

tions are handled properly, there are some limitations to its use.  Perhaps

the most important of these is that, even in the "free-run" mode, execution

is quite slow compared to an actual 6502 running at normal clock speeds.  No

attempt has been made to correlate execution times with normal 6502 clock

periods.  (This may, in fact, be impossible due to the way in which the sys-

tem handles multiple addressing modes.)  Therefore, programs incorporating

software timing loops will not execute properly.  Also, since the package was

intended for software development, hardware interrupts are not supported, and

only a limited amount of hardware interfacing can be performed through the

"user" subroutine calls.

Two program listings are included here.  The first, called "ZXLD",

is a short program whose only function is to boot the main program from the

CP/M Transient Program Area to high memory.  The second listing is the in-

terpreter itself, assembled to reside from 6F00 to 7DFF (HEX) in a 32K sys-

tem.  Note that this leaves 512 bytes, normally containing CBIOS, unaffected.

ZX65: Simulating a Micro					Page 9

The program is not relecatable, so that in order to run in a different mem-

ory configuration, it will be necessary to reassemble the source with a new

value in the "SIZE EQU XX" statement.  All other addresses are derived from

this value.  If the object code is to be manually loaded for the 32K system

then "ZXLD" must be loaded first, starting at 0100 (HEX).  Then start loading

ZX65 at 0110 (HEX), continuing until the end.  Finally, use the CP/M "SAVE"

command (SAVE 15 ZX65.COM) to write the object code to disk.  The package can

now be invoked by answering "ZX65" to the CP/M prompt.  If you are reassem-

bling the package, then you should also reassemble the ZXLD routine (modified

for your system) so that you have a .HEX file for each module.  Then load

the package using DDT as follows:

DDT ZXLD.HEX

IZX65.HEX

Rdisp

where "disp" is the load offset value required to load the object code from

the .HEX file at a different address than that specified.  (DDT will calcu-

late "disp" for you if you type "H110,ssss", where "ssss" is the actual

starting address of your program.  DDT will respond with the sum and diff-

erence of the two values, and the difference will be the required value

"disp".)  Finally, save the object code as directed above for manual loading.

ZX65: Simulating a Micro					Page 10

To run the program, place the disk with ZX65.COM on drive A (this

disk should also have a copy of CP/M so that the system can be restarted, if

necessary, without changing disks.)  Place the ZX65 data disk on drive B.

Start CP/M and invoke ZX65 by answering "ZX65" to the CP/M input prompt.

The system will prompt you with a header and a ">" symbol.  You are now in

the command mode, and typing one of the command codes (see table 4) will

cause the appropriate action to occur.  Note that if you are using a disk

not previously used for ZX65 on drive B, you will be unable to use the ZX65

DOS functions until you have initialized the disk.  This is done simply by

typing "I", at which point the system will ask you to verify this request

with a "Y" or "N".  The reason for this (note this well) is that the "I"

command clears and initializes the directory on disk B.  This is the equiv-

alent of the CP/M command "ERA *.*" and must be used with caution.

For those unwilling to type in a 4K byte program by hand, I will

make the package available on a standard 8 inch IBM format, single side,

single density diskette (Sorry, I can't handle any other format!) at a nom-

inal charge of $15 to cover time and material.  Write to me at the address

given at the head of this article, and specify your system memory size.  I

will send a diskette containing both source and object files assembled to

reside in high memory as described above, as well as a copy of ZXLD.

ZX65: Simulating a Micro					Page 11

A command summary, operating hints, and other general information will be in-

cluded on the disk.

Finally, although this program has been rather exhaustively tested

with a variety of 6502 software (both simple and sophisticated) I cannot

guarantee it bug-free.  I have included no copyright notices of any kind,

and I hope users will feel free to copy, add, delete, and improve at will.

(I would like to hear of any major modifications...I may want to include them

myself!)  As stated at the outset, the package was developed as a side benefit

of another project, not as a money maker, and if anyone else finds it useful,

so much the better.				-RMK

Acknowledgements:

*CP/M is a registered trademark of Digital Research, Pacific

Grove, CA.

While not specifically mentioned in the text, I have relied

heavily on the information contained in the book "6502 Assembly

Language Programming" by Lance A. Leventhal (Osborne/McGraw Hill

Inc., 1979)

##

#orne/McGraw Hill

Inc., 1979)

#

#

##�#�#�#�#�#�#�#�#�#�#�#�#�#�#�#�#�#�#�#�#�#�#�#�