*****	Notes on ZX65R v 2.1 and ZXLDR v 2.1	9/80	RMK

The ZX65R program and source listings on this disk represent a

substantial improvement over those published in DDJ #47.  The 'R' in the

program names means 'relocatable', and that means that the same ZX65

.COM file will now run on any size CP/M machine (except Heath/Zenith,

TRS80, and any others using odd memory configurations).  This has been

accomplished mainly through a new loader, ZXLDR.  This loader contains

a complete relocation map for ZX65R.  It finds your CBIOS by examining

the warm start vector at 0001H.  It then insinuates ZX65 just below CBIOS,

after first calculating and inserting new values for all non-relocatable

operands in the 'prototype' ZX65R code.  The loader and the main program

appear as two separate source modules which have been concatenated using

a linking loader, resulting in the run-able ZX65.COM file.  To run the

program, transfer ZX65.COM onto a CP/M system disk using PIP.  Boot up

this disk in drive A, and place a blank disk that you want to use for

6502 programs and data in drive B.  Type 'ZX65'.  Drive B will operate

briefly and the ZX65 prompt will appear on the console.  You are now in

ZX65's command mode.  Refer to the Dobb's article or file ZXTAB1.DOC for

a command summary.

If you compare the source code for ZX65R with that published

in DDJ, you will notice what appear to be substantial differences in the

listing.  The most obvious change is in the header comments and the 'eq-

uates' which set up memory locations for the system. Even though this

looks quite a bit different, it is for the benefit of relocatability and

has no effect on the operation of the program once it is loaded.  If you

will look a little further and compare the actual instructions, you will

see that they are mostly unchanged.  There are two exceptions. The first

involves the code from line 339 to line 428. These are the instruction

processors for JMP, JSR, RTS, and RTI, and the new code corrects for im-

proper stack handling in the original version. The second change is down

near the end of the code in the console I/O handler. A patch there accom-

odates a few BIOS console output routines which do not return with the

output character still in the accumulator.

Notice that the memory locations for the USER subroutine link-

ages shown in Table 3 apply only after loading is complete.  To insert

your own subroutine addresses, the best method is to load ZX65.COM under

DDT. You will then find the USER jump table at the following locations:

USR0		0FCCH *		* The first two locations are

USR1		0FCFH *		initialized to console I/O and

USR2		0FD2H 		will be relocated during load-

USR3		0FD5H		ing. The last three will not

USR4		0FD8H		be relocated.

You may also wish to change the 16-bit 'BRK' vector:

BRK VECTOR	0FDBH		Not relocated during loading.

Once you have inserted the desired changes under DDT, you may either ex-

ecute the program directly by typing G100, or save it to disk following

warm start with the command 'SAVE 20 ZX65.COM'.

CP/M COMPATABILITY: As of this writing, I have not yet succum-

bed to Digital Research's efforts to get me to buy the new CP/M 2. From

what I have been able to figure out, it offers very little to me in the

way of advantages. Suffice it to say that ZX65 was written and debugged

using V 1.4, and I cannot say whether or not it will run under any other

version. (My guess is that it will work with V 1.3, but not v 2.X.) The

main reason for any incompatibility will probably have to do with the

fact that my self-contained mini DOS operates the disks and console dir-

ectly through CBIOS rather than by using calls to BDOS which is the more

conventional method. (There is a good reason for this madness which I

will explain shortly.) In any event, since you have spent your hard-

earned nickels for this copy, I suspect that you are resourceful enough

to overcome such problems and implement any necessary patches. If you

do come up with a running version patched for CP/M 2 or for some other

disk format (i.e. North Star etc.), how about submitting it for possible

publication in Dobb's? (No, I'm not on their payroll, but they are re-

ceptive to this type of material and lots of 'hard core' dedicated

computerists read their mag.)

SOME NITTY-GRITTY: As promised, here is the explanation of why

I have interfaced through CBIOS rather than using BDOS: If you have

done any work at all with the 6502 processor, you will recall that page

zero of memory (0000H to 00FFH) is sacred to it. There are short forms

of many instructions for use on page zero, and the very powerful indirect

indexing modes always use page zero. Now ZX65 by design uses no memory

mapping at all...that is, if your 6502 program is ORG'd at 2000H, that

is exactly where you will load it. (This is in contrast to at least one

TRS80-based simulator that I know of.) Having gone this far, it is only

reasonable to expect that page zero will be totally available to the

6502 program. As you have probably realized by now, the catch here is

that BDOS also likes page zero, and uses large chunks of it during disk

accesses. With one possible limitation (which I will discuss in a bit)

ZX65 totally removes the host system from page zero...even the vectors

at the bottom of memory need not be saved.  ZX65 is, in fact, confined

to the top 1100H bytes of user memory (more if your CBIOS is longer than

512 bytes), and ALL space below that is available for 6502 programs and

data. The one possible conflict that I mentioned is system-dependent,

and may or may not apply in your case. CP/M V 1.4 has a reserved 'scr-

atch' area of 16 bytes located from 0040H to 004FH. Some disk controllers

use this area for temporary data storage during all disk transfers, whe-

ther initiated by CP/M or not. If this is true of your controller, then

you have very little choice but to accept the fact that this portion of

page zero will be messed up whenever you do a disk access from ZX65.

There is still one alternative (I know because I did it!): you may be

able to move this scratch area out of page zero into high memory. Even

if you have this conflict, 6502 programs that use page zero only for

temporary data storage will probably run normally unless the 6502 pro-

gram itself accesses the disk. (Yes, it is possible, using the 'system'

subroutine calls.)

ZXDOS COMMENTS: Having destroyed BDOS as described above, I was

obliged to include an embedded disk operating system in ZX65. Now, ZXDOS

will never win any awards for versatility, but it is, I think, adequate

for the application. All internal disk access is confined to drive B (a

two drive system is assumed). ZXDOS files are not CP/M compatible. A dir-

ectory is maintained on track zero, and the remainder of the disk is

available for program/data storage. Each directory entry is 16 bytes long,

and contains the file name (format same as CP/M but file type has no spe-

cial meaning), the load address, the disk location, the number of sectors,

and an active/dead flag byte. During a directory display or search, the

directory is paged through a buffer within ZX65 one sector at a time,

continuing until an 'end of directory' mark is located. Files are stored

sequentially and on sequential sectors (no interleaving). A new file will

normally be appended at the tail end of existing files, unless the entire

file will fit into the space allocated for a dead file. Thus a rudimentary

form of dynamic disk space allocation is used, but a full disk will still

contain quite a few empty slots. Killing a file does not actually erase

anything, but simply resets the active status flag in the directory for

that entry. The 'initialize' command writes a properly formatted empty

directory to the disk.

Finally, it is be possible to use ZX65 with only one drive. To

implement this, load ZX65.COM using DDT, and change the value at address

484H from '01' to '00'.  Save the modified code back to a system disk

following warm start using a 'SAVE 20 ZX65.COM' command. To operate in

this mode, first start ZX65, then immediately remove the system disk

and insert the ZX65 data disk.  From this point operation will be the

same. This works because once loaded, ZX65 does not itself require any

further disk access.

CONCLUSION: Have fun with ZX65!		-RMK

###e

ZX65 data disk.  From this point operation will be the same. This works

because once loaded, ZX65 does n��������