Microcosm Associates CPU Test/Diagnostic

for the

8080 and 8085

Donated to the "SIG/M" CP/M User's Group by Kelly Smith

This is an extensive diagnostic for the 8080 and 8085 CPU, and

exercises  most functions of the processor.  This diagnostic is

NOT for the Z80,  but may be modified as required to handle the

incompatible  "Sign/Parity  Flag  Bit".   This  diagnostic  was

especially helpful in tracking down problems I encountered with

early production run's of the NEC 8080 which exhibited problems

with the "Aux Carry Flag Bit" when doing "DAA's".

A  succesful  run of the program will  display  the  following

message:

CPU IS OPERATIONAL

an  unsuccesful run of the program will display the  following

message:

CPU HAS FAILED!   ERROR EXIT=NNNN

Where  "NNNN" is the address location AFTER the test  failure.

The  CPUDIAG.PRN file may then be referenced as to the  address

location  "EXIT" of the failure,  to determine the exact nature

of the problem. Error address's were not included in the ".DOC"

file,  because  the program is typically relocated in PROM  for

system  diagnostic  purposes.  The  program  has  however  been

CP/Mified to run as a ".COM" file,  with BDOS calls for console

I/O.

This  program  also serves as a very good "tutorial aid"  when

traced  with Digital Research's DDT or SID,  to gain  a  better

understanding of register manipulation and arithmetic processes

of the 8080 and 8085 CPU.

Kelly Smith, Microcosm Associates

3055 Waco Avenue

Simi Valley, California, 93065

(805) 527-9321 (Modem, CP/M-NET)

(805) 527-0518 (Verbal)

######################################################