TERMINAL INSTALLATION

Before you use PATCH, it must be installed to your particular terminal

i.e., provided with information regarding control characters required

for certain functions.	This installation is easily performed using the

program PINSTALL which is described in the following.

After having made a work-copy, please store your distribution diskette

safely away and work only on the copy.

Now start the installation by typing PINSTALL at your terminal.  Select

sreen installation from the main menu.	Depending on your version of

PATCH, the installation proceeds as described in the following two sec-

tions.

A menu listing a number of popular terminals will appear, inviting you

to choose one by entering its number:

------------------------------------------------------------------------

Choose one of the following terminals:

1) ADDS 20/25/30	       15) Lear-Siegler ADM-31

2) ADDS 40/60	       16) Liberty

3) ADDS Viewpoint-1A      17) Morrow MDT-20

4) ADM 3A		       18) Otrona Attache

5) Ampex D80	       19) Qume

6) ANSI		       20) Soroc IQ-120

7) Apple/graphics	       21) Soroc new models

8) Hazeltine 1500	       22) Teletext 3000

9) Hazeltine Esprit       23) Televideo 912/920/925

10) IBM PC CCP/M b/w       24) Visual 200

11) IBM PC CCP/M color     25) Wyse WY-100/200/300

12) Kaypro 10	       26) Zenith

13) Kaypro II and 4        27) None of the above

14) Lear-Siegler ADM-20    28) Delete a definition

Which terminal? (Enter no. or ^X to exit):

------------------------------------------------------------------

Terminal Installation Menu

If your terminal is mentioned, just enter the corresponding number, and

the installation is complete.  Before installation is actually perform-

ed, you are asked the question:

------------------------------------------------------------------

Do you want to modify the definition before installation?

------------------------------------------------------------------

This allows you to modify one or more of the values being installed as

described in the following.  If you do not want to modify the terminal

definition, just type N, and the installation completes by asking you

the operating frequency of your CPU (see last item in this appendix).

If your terminal is not on the menu, however, you must define the re-

quired values yourself.  The values can most probably be found in the

manual supplied with your terminal.

Enter the number corresponding to "none of the above" and answer the

questions one by one as they appear on the screen.

In the following, each command you may install is described in detail.

Your terminal may not support all the commands that can be installed.

If so, just pass the command not needed by typing RETURN in response to

the prompt.  If Delete line, Insert line, or Erase to end of line is not

installed, these functions will be emulated in software, slowing screen

performance somewhat.

Commands may be entered either simply by pressing the appropriate keys

or by entering the decimal or hexadecimal ASCII value of the command.

If a command requires the two characters 'ESCAPE' and '=', may:

either Press first the ESC key then the =.  The entry will

be echoed with appropriate labels i.e., <ESC> =.  Enter the

decimal or hexadecimal values separated by spaces.  Hexadec-

imal values must be preceded by a dollar sign.	Enter e.g.

27 61 or $1B 61 or $1B $3D which are all equivalent.

The two methods cannot be mixed i.e., once you have entered a non-numeric

character, the rest of that command must be defined in that mode, and

vice versa.

A hyphen entered as the very first character is used to delete a com-

mand, and echoes nothing to the text.

Terminal type:

------------------------------------------------------------------------

Enter the name of the terminal you are about to install.  When you com-

plete PINSTALL, the values will be stored and the terminal name will ap-

pear on the initial list of terminals.	If you later need to re-install

PATCH to this terminal, you can do that by choosing it from the list.

Send an initialization string to the terminal?

------------------------------------------------------------------------

If you want to initialize your terminal when  PATCH starts (e.g. to down-

load commands to programmable function	keys), you answer Y for yes to

this question.	If not, just hit RETURN.

If you answer Y, you may choose between entering the command directly or

defining a file name containing the command string.  The latter is a

good idea if the initialization string is long, as sinc a string to pro-

gram a number of function keys would be.

Send a reset string to the terminal?

------------------------------------------------------------------------

Here, you may define a string to be sent to the terminal when PATCH ter-

minates.  The description of the initialization command above, applies

here.

CURSOR LEAD-IN command:

------------------------------------------------------------------------

Cursor Lead-in is a special sequence of characters which tells your ter-

minal that the following characters are an address on the screen on which

the cursor should be placed.  When you define this command, you are asked

the following supplementary questions:

--------------------------------------------------------------------

Some terminals need a command between the two numbers defin--

ing the row and column cursor address.

CURSOR POSITIONING COMMAND to send after line and column:

-----------------------------------------------------------

Some  terminals need a command after the two numbers defin-

ing the row and column cursor address.

Column first?

-----------------------------------------------------------

Most terminals require the address on the format:  First

ROW, then COLUMN.  If this is the case on your terminal,

answer N.  If your terminal wants COLUMN first, then ROW,

then answer Y.

OFFSET to add to LINE

-----------------------------------------------------------

Enter the number to add to the LINE (ROW) address.

OFFSET to add to COLUMN

-----------------------------------------------------------

Enter the number to add to the COLUMN address.

Binary address?

-----------------------------------------------------------

Most terminals need the cursor address sent on binary form.

If that is true for your terminal, enter Y.  If your termi-

nal expects the cursor address as ASCII digits, enter N.

If so, you are asked the supplementary question:

2 or 3 ASCII digits?

-----------------------------------------------------------

Enter the number of digits in the cursor address for your

terminal.

CLEAR SCREEN command:

------------------------------------------------------------------------

Enter the command that will clear the entire contents of your screen,

both foreground and background, if applicable.

Does CLEAR SCREEN also HOME cursor?

------------------------------------------------------------------------

This is normally the case; if it is not so on your terminal, enter N and

define the cursor HOME command.

DELETE LINE command:

------------------------------------------------------------------------

Enter the command that deletes the entire line at the cursor position.

INSERT LINE command:

------------------------------------------------------------------------

Enter the command that inserts a line at the cursor position.

ERASE TO END OF LINE command:

------------------------------------------------------------------------

Enter the command that erases the line at the cursor position from the

cursor position through the right end of the line.

START OF 'LOW VIDEO' command:

------------------------------------------------------------------------

If your terminal supports different video intensities, then define the

command that initiates the dim video here.  If this command is defined,

the following question is asked:

START OF 'NORMAL VIDEO' command:

------------------------------------------------------------

Define the command that sets the screen to show characters

in 'normal' video.

NOTE:  Because PATCH uses REVERSE VIDEO HIGHLIGHTING, you may

need to REVERSE the definition of 'LOW VIDEO' and 'NORMAL VIDEO'

in order to get the proper effect.  When properly installed, the

main menu should have a HIGHLIGHTED boarder with the required

input character for each command HIGHLIGHTED.

Number of rows (lines) on your screen:

------------------------------------------------------------------------

Enter the number of horizontal lines on your screen.

Number of columns on your screen:

------------------------------------------------------------------------

Enter the number of vertical column positions on your screen.

------------------------------------------------------------------------

Delay after CURSOR ADDRESS (0-255 ms):

Delay after CLEAR, DELETE, and INSERT (0-255 ms):

Delay after ERASE TO END OF LINE and HIGHLIGHT On/Off (0-255