Turbo Pascal Communications Library

1.0 Introduction

Turbo   Pascal   provides   a   fairly   uniform  programming

environment for a variety of microcomputers  running  CP/M  and

MS-DOS.   It is fast, and the differences between versions can,

in general, be worked around.  Certain  areas  are  a  problem,

however - communications, and system dependent function.

This  set  of  routines  is  intended to address the first of

these  problems, difficulty in  communicating with  the outside

world.  Each   operating  system  is  different in  the way  it

handles communications, and different hardware  implementations

create problems when trying to use communications ports.  There

is a need for a standard set of routines that provide the basic

communications  needs  for  most programs.  If the routines are

rewritten  for  various  micros,   generalized   communications

programs can be written.  Any programming tricks can be used to

perform  the  actual  functions, but a uniform calling sequence

can be used.

1.1 Sample Programs

A   sample   communications   library   is  available  as

'CLIB180.INC'.  This library includes an  example  of  each

function and procedure listed below.  Some of the functions

are  very  simple, as the VT180 has PSETUP available to set

parity and/or baudrates.  CLIB180 also supports the RAINBOW

(100A,100B) running CP/M version 2 or higher.

A  sample  of  the  use  of the communications library is

given  in  the   program   'COMDEMO.PAS'.    This   program

implements a 'connect' mode and allows simple file transfer

by  capturing data from the communications port and storing

it in a file, or sending a file to a remote system.

COMDEMO.PAS  was written as a 'test-case' for CLIB180.INC

- it's also useful as the "first piece"  to  "customize"  a

"text-capture"   utility  for  a  new  micro-system,  which

doesn't have yet a way to "talk protocol".   TURBO-Pascal's

inherent  compilation-speed  and  (relatively  high  -  see

above) portability will help in  getting  the  "first"  and

"crucial"  step accomplished : Enable TEXT-capture "to" and

"from" another system.  The typical "next step" seen is  to

establish  an  (preferably)  Error-Correcting protocol like

KERMIT.

2.0 General Library Description

2.1 Variables, Constants, and Typed Variables

All  library  functions,  procedures, and global variables

begin with 'c_'.  This should minimize collisions with other

program variables.  Any variables that begin with 'c__'  are

for  the  use  of  the  library  itself, and should NEVER be

modified by the main program.

All  variables  required  by  the  library  only should be

included here.  This would include such variables as  IOBYTE

definitions for CP/M, etc.

The   following   are  predefined  types,  constants,  and

variables in the library:

Types

c__string10 = string[10];

c__string5 = string[5];

c__port_names = an  array  of string10's that will list the

possible ports for the computer.

c__baud_names = an  array  of string10's that will list the

possible baud rates for the computer.

c__parity_names = an  array  of string10's that will list the

possible parity settings for the computer.

Constants

c_lib_version = computer type, revision of library, and date.

c_port_str = all possible  strings that  the available  ports

are  equivalent  to.   The  first  two  MUST  be

'NOT SET'  and  'DEFAULT'.  After  all ports are

defined, the last  element in the  array MUST be

null ('').

c_baud_str = all possible strings that the available baud rates

are equivalent to.  The first two MUST be

'NOT SET' and 'DEFAULT'.  After all baud rates are

defined, the last element in the array MUST be

null ('').

c_parity_str = all possible strings that the available  parity

settings are equivalent to.  The first two MUST

be  'NOT SET' and  'DEFAULT'.  After  all ports

are defined, the last element in the array MUST

be null ('').

Variables

c_kbd_char : The returned keyboard character.

c_comm_char : The returned comm port character.

c_current_port : The number of the current port. the string

equivalent of the port number is :

c_port_str(c_current_value).

c_current_baud : The number  of the current baud rate.  The

string equivalent of the baud rate is :

c_baud_str(c_current_baud).

c_current_parity : The number of the current parity setting.

The string equivalent if the parity is :

c_parity_str(c_current_parity).

2.2 Functions

2.2.1 C_INIT

Initialize  a  communications port.  Initialization is

system dependent,  but  should  include  such  items  as

setting  hardware specific values, obtaining the initial

value of the IOBYTE in CP/M systems, etc.  It should  be

run  at  the  beginning  of  each  program that uses the

communications library.

The way the port, baud rate, and parity variables are

set up, executing c_init(1,1,1) will set up a default

communications port.  This is a good way to begin any

program so the at least some communication port is

initialized.  For some systems, having no port set up

will cause system hangups.

c_init(port, baud, parity : integer) : boolean;

Inputs : port   -  Number  of  the  port.  This  can  be

obtained  by  scanning  the  list  of

ports  contained    in   the    array

c_port_str.

parity -  desired   parity   setting  on  port.

The number  of the  available  parity

values  for the  particular computer.

These can be obtained by scanning the

list of parity  settings available in

the array c_parity_str.

speed -   desired  port  speed.  The number of

the available baud rate settings can

be obtained by scanning  the list of

available  baud rate settings in the

array c_baud_str.

Output : Function  returns TRUE if the desired setup  is

valid, and FALSE if the setup is invalid.

Variables affected : c_current_port, c_current_baud, and

c_current_parity are set to their new

values if the operation was successfull.

Other effects: depends on implementation.

2.2.2 C_SET_PORT

Select the desired communications port.

c_set_port(port : integer) : boolean;

Input : port - Number of the port.  System dependent.  A

list of the available ports  can be found

in the array c_port_str.

Output : Function returns TRUE if new port was selected,

and FALSE if the new port was an invalid port.

Variables affected : sets c_current_port, c_current_baud,

and c_current_parity to those for the

new port.  Other variables are system

dependent.

Other  effects  :  current port should not be changed if

function returns FALSE.

2.2.3 C_SET_PARITY

Set the parity on the current port, if possible.

c_set_parity(parity : integer) : boolean;

Input : port - Number of the port.  System dependent.  A

list of the available ports  can be found

in the array c_port_str.

parity -  desired   parity   setting  on  port.

The number  of the  available  parity

values  for the  particular computer.

These can be obtained by scanning the

list of parity  settings available in

the array c_parity_str.

Output: Function  returns TRUE if the parity is reset to

the new value, and FALSE if the parity could not

be set.

Variables affected: c_current_parity is changed to the new

parity if the operation was successfull.

Other variables are system dependent.

Other effects: should leave same parity if result is FALSE.

2.2.4 C_SET_BAUD

Set the baud rate on the specified port, if possible.

c_set_baud(speed : integer) : boolean;

Inputs : speed -   desired  port  speed.  The number of

the available baud rate settings can

be obtained by scanning  the list of

available  baud rate settings in the

array c_baud_str.

Output: Function returns TRUE if the baud rate can be

set and FALSE if the baud rate cannot be set.

Variables Affected: c_current_baud is set to the number

of the new baud rate if the operation

successfull.  Other variables are

system dependent.

Other  effects  : Should leave the baud rate the same if

error.

2.2.5 C_GET_KBD_CHAR

Check the keyboard for an available character.

c_get_kbd_char : boolean;

Input : none.

Output : Function   returns   TRUE  if  a  character  is

available  and  false  if   no   character   is

available.

Variables affected : c_kbd_char   is   changed   to  new

character if function returns true.

Other effects : function  must always leave the computer

in  the same state as it was found, i.e.

for  CP/M the IOBYTE must be returned so

that  CON: is set to main console.

2.2.6 C_GET_COMM_CHAR

Check  the currently selected communications port for an

available character.

c_get_comm_char : boolean;

Input : none.

Output : Function   returns   TRUE  if  a  character  is

available  and  false  if   no   character   is

available.

Variables affected : c_comm_char   is   changed  to  new

character if function returns true.

Other effects : Function  must always leave the computer

in  the same state as it was found, i.e.

for  CP/M the IOBYTE must be returned so

that  CON: is set to main console.

2.3 Procedures

2.3.1 C_PUT_SCR_CHAR

Send a character to the console screen.

c_put_scr_char(character : char);

Input : character to be output.

Output : none.

Variables affected : none.

Other effects : Function  must always leave the computer

in  the same state as it was found, i.e.

for  CP/M the IOBYTE must be returned so

that  CON: is set to main console.

2.3.2 C_PUT_COMM_CHAR

Send a character to the selected communications port.

c_put_comm_char(character : char);

Input : character to be output.

Output : none.

Variables affected : none.

Other effects : Function  must  always leave the computer

in  the same state as it was found,  i.e.

for   CP/M the IOBYTE must be returned so

that  CON: is set to main console.

3.0 VT180 Communications Library

This  is  an implementation of a standard set of routines for

communications, for the DEC  VT180  (Robin).   The  VT180's

main features are as follows:

Microprocessor:         Z80A

Memory:                 64k bytes

Operating system:       CP/M-80

Communications ports:   4 serial

CRT:  hard-wired terminal port to VT100 terminal

LPT:  printer port

UC1:  user defined console port

PTR:  high speed reader port

PTP:  high speed punch port

The  VT180  uses  the Digital Research standard IOBYTE for port

switching.

3.1 The IOBYTE

A few words about the IOBYTE are in order.  The ports on

a  CP/M-80  computer are described by PHYSICAL names (CRT:,

LPT:, etc.).  When the IOBYTE is used, they are  mapped  to

LOGICAL  ports,  and  the IOBYTE allows reassignment of the

physical ports to different logical ports.  For example, on

the VT180, the pysical ports UC1: and CRT: can be  assigned

to  be  the  logical  port  CON:.  When booted normally all

console I/O goes to CRT:,  the  internal  VT100  port.   If

desired,  the  UC1:  port can be assigned to be CON: and an

external device can act as the system console.  I use  this

method  with  a  modem so I can dial up my computer and use

the remote terminal as the console.

The IOBYTE is the device that allows this reassignment of

physical  ports.   The IOBYTE is a byte in memory, normally

located  at  address  0003.   This   may   vary   in   some

implementation  of  CP/M.   There  are  four  fields to the

IOBYTE:

LIST       PUNCH      READER       CONSOLE

bits 6,7   bits 4,5    bits 2,3      bits 0,1

The values below are the possible IOBYTE assignments:

Console field (bits 0,1)

00 console is assigned to the console printer device (TTY:)

01 console is assigned to the CRT device (CRT:)

10 batch mode, RDR: is console input device, LST: device is

console output.

11 console is assigned to the user defined console (UC1:)

READER field (bits 2,3)

00 reader is teletype device (TTY:)

01 reader is the high speed reader device (PTR:)

10 reader is user defined reader #1 (UR1:)

11 reader is user defined reader #2 (UR2:)

PUNCH field (bits 4,5)

00 punch is teletype device (TTY:)

01 punch is high speed punch device (PTP:)

10 punch is user defined punch #1 (UP1:)

11 punch is user defined punch #2 (UP2:)

LIST field (bits 6,7)

00 list device is teletype (TTY:)

01 list device is CRT device (CRT:)

10 list device is lineprinter device (LPT:)

11 list device is user defined list device (UL1:)

Two  of  the  serial  ports on the VT180 can be used for

serial communication, the READER/PUNCH port,  and  the  UC1

(user defined console) port.  From the chart above, setting

the  two  least  significant  bits of the IOBYTE to 01 will

allow normal console I/O to the  VT100  terminal.   Setting

the  same two bits to 11 will allow the UC1: port to be the

system console.

Working   with   the   READER/PUNCH  port  is  a  little

different.  This is because the status of the  reader  port

cannot be obtained directly with reader system calls.  If a

call to get character from reader is done, the routine will

hang  there  until a character is available.  This will not

allow the alternating status checking  required.   However,

there  is  a  way.  The reader port can be made the console

port (IOBYTE value xxxxxx10).  Now the status of the reader

can be obtained without waiting.  The  characters  received

from  the keyboard are sent to the punch port with a system

call to punch output.

2.5.3 Eight Bit Data

The  Digital  Research  manual for CP/M specifies that

the hight order bit of a  character  obtained  with  the

CONIN bios call will be zero.  This is not the case with

the  VT180.   All  8  bits are significant.  This allows

binary data to be transmitted and received by the VT180.

Other systems may vary.

2.5.4 Function Descriptions

2.5.4.1 Function C_INIT

The c_init function sets up the  intial values for

the communications  library.  This function  MUST be

executed at the beginning  of any program  that will

use  the comm  library.  It initializes  the  iobyte

settings, and sets up the default values for all the

possible ports.

On  entry, it sets all  the ports to their default

values.  Since the parity and baud rate on the VT180

cannot  be  easily  set,  DEFAULT  is  all  that  is

allowed  for these  settings.  The default  port  is

the comm port, which  is the normal VT180 comm port.

If  an  illegal  port,  baud  rate,  or  parity is

requested, the function returns FALSE.

2.5.4.2 Function C_SET_PORT

Two ports are allowed for the VT180.  Valid values

for port are 1, 2, and 3,  yielding default  (COMM),

COMM, and UC1, respectively.

2.5.4.3 Function C_SET_BAUD

In  this  version of the library, baud rates cannot

be  set, and  the function  will return FALSE  if any

value other than 1 (default) is requested.

2.5.4.4 Function C_SET_PARITY

In  this  version  of the library, parity cannot be

set, and  the  function  will  return FALSE  if any

value other than 1 (default) is requested.

2.5.4.5 Function C_GET_KBD_CHAR

This  function uses bios(1) to obtain  keyboard

status  and  bios(2)  to  obtain a character.

2.5.4.6 Function C_GET_COMM_CHAR

This  function uses bios(1) to obtain port status

and bios(2) to obtain a character.

2.5.5 Procedures

2.5.5.1 Procedure C_PUT_SCR_CHAR

This procedure uses bios(3) to send a character

the screen.

2.5.5.2 Procedure C_PUT_COMM_CHAR

This  function  checks whether the selected port is

the reader or the UC1 port and uses bios(5) to  write

to the punch port or bios(3) to write to UC1.

####################################################################