RS232 CABLING            Last Update:

Jan 05, 1985

This is a brief discussion of experiences in connecting computer

devices with RS232 (serial interface) cables. The level of

discussion is directed to being able to generally hook up a good

serial printer to different personal computers. While the

discussion should apply generally, the specific computers used

are a CPM machine (Osborne1) and an IBM-compatible (Eagle Spirit

XL). The output devices used are a Centronics 704 printer, a NEC

3510 Spinwriter, a Hayes Smartmodem, a Strobe paper plotter, and

a Houston Instruments paper plotter. In addition, hooking

computers directly to each other will be added later (I hope).

In addition to the "wiring configuration" issues discussed here,

many computer devices have options set by program logic, system

logic, or switches that must be matched across connected devices

before correct operation. Consult your users guide and other

sources of reference material for other specification types.

Often, computer devices connected together act like nothing is

right until every specification is correct. Another common case

(especially when parity or bit speed specifications don't match)

is for garbage characters to appear as transmitted data.

**********Copywrite*********

This material is copyrighted by Global Data Processing

of Dallas, Texas. Permission for non-commercial use, such

as computer bulletin board uploading/downloading is granted.

Any commercial use or inclusion must have written permission.

Contact Global Data Processing for written permission: (214)

243-8744.

**********Copywrite**********

**********Cautions:**********

No warranty or suitability of use or responsibility is assumed

nor implied by the author(s) or publishers or repository agents

of this write-up.

**********Cautions:**********

Mis-cabling or incorrect connections can cause permanent damage

to equipment, requiring expensive repairs or replacement. This

damage typically is not covered by equipment warranties. I now

own a dead printer, ruined by a mis-connection.

**********Cautions:**********

While there is specifically-designed test equipment that permits

signal lights and voltage level displays, ordinary voltmeters or

signal lights can cause permanent damage to computer equipment.

For example, current drain can be too heavy by some signal

lights that use your computer equipment signals to illuminate the

lights.

When I am figuring out a new cable problem, I borrow a

professional break-out box which does not drive signal lights

from the computer device voltages/current (it detects signals

and has batteries to light the signal lights).

**********Cautions***********

INTRODUCTION

The first discussion covers a critical conceptual point:

--RS232 configuration type (defined by the equipment vendor)

RS232 wire functions (what do all those wires do?) is a topic

to be added later, I hope. Also note that the data-sending

logic used is largely independent of specific RS232 cabling

decisions. That is, whether 7 or 8 bit characters are sent,

or EVEN/ODD/NO parity characteristics are used is a different

equipment-related decision than the specific cable wiring

configuration.

The discussion here presents only specific cable wiring

experiences with diagrams. A final note is that Figure 5 is a

abnormal and very simplistic modem hook-up. Many modems and many

modem programs will not work with a "bare minimum" connection as

shown in Figure 5.

CABLE WIRING: CONFIGURATION TYPES

When an equipment vendor decides to provide an RS232 interface

(or port), a decision has to be made on which configuration

type is to be used. The equipment can be defined as primarily

SENDING TO terminal equipment, thus acting as Data Communications

Equipment (DCE). Or the equipment can be manufactured as

primarily RECEIVING FROM controlling communications devices,

thus being Data Terminal Equipment (DTE). Several pairs of

wires (or pins) in the plug can be reversed depending on the

specific manufacturer-defined configuration type.

Configuration type is important because connecting DCE (such

as Osborne 1) to DTE (all serial printers and plotters that I

have encountered) is usually easy, with a simple cable.

However, hooking up DTE (IBM-compatibles) to DTE (serial printers,

etc.) requires specially-made cables (typically reversing 1 or

more pairs of cables). Overall, the difficulty is that hooking DCE

equipment to DCE equipment, or DTE equipment to DTE equipment

requires reversing 1 or more pairs of cables, or requires a

"null modem" cable (see Dennis Recla's write-up).

A common point of confusion in discussing a particular device

are the directional words such as "TRANSMITTED DATA PIN",

"RECEIVED DATA PIN", etc. While pin 2 is usually identified as

"TRANSMITTED DATA", it only is such on DTE equipment. On DCE

equipment, "TRANSMITTED DATA" (that is, transmitted from the

"host" (DCE) to the terminal/printer (DTE) is pin 3. Yet vendors

sometimes document their configuration without a directional

notation or a configuration type notation.

The confusion arises from the failure to clearly identify

equipment as DTE or DCE. That is, the description of single

directional signals such as "TRANSMIT DATA PIN" needs to

include a "TO THIS DEVICE" or "FROM THIS DEVICE" notation.

My references (Centronics, NEC Spinwriter, Strobe , and

Houston Instruments technical manuals) all include directional

notations.

Since directional signals are relative to configuration, you

must typically identify what relative configuration types

you have in equipment that you want to hook up. If the

devices are the same type, you must have a "non-standard"

cables. Unless the vendor has some customized pin usage or

other restriction, standard RS232 cabling should be usable

for hooking up DTE equipment to DCE equipment. (There are

restrictions such as limit on cable length, since RS232

cables easily pick up electrical noise from the environment

or from its own closely adjacent wiring. Also see Figure 6 (the

Houston Instruments plotter) for a non-standard requirement.

INTERFACE FUNCTIONS

Another decision that has to be made by the vendor is to define

which functions will be implemented in the interface. The

simplest (and cheapest) interfaces use only 3 active wires, while

a "good functionality" interface may use 10 active wires.

Typically the connections being made are between computers and

printers/modem devices. The functions performed across the RS232

cable interface commonly are on individual wires and usually

include:

--signal ground (i.e., a zero reference voltage)

--a transmit signal

--a receive signal

(the first three are almost always there)

--frame ground ("grounded" protection)

--data set ready ("transmit device" power is on)

--data terminal ready ("receive device" power is on)

--ready to send (now have data to send)

--clear to send (am ready to receive data)

This write-up will not go into the functional descriptions, other

than the above brief identification.

FIGURES

The following figures diagram the cabling ("pin-outs") used

in several specific computer device scenarios.

FIGURE 1      Osborne 1 connected to Centronics 704

Osborne 1 (DCE)                  Centronics 704 (DTE)

PIN                               PIN

(terms relative to DTE)

1----------frame ground-----------1

2<----------transmit data---------2

3-----------receive  data-------->3

4<----------request to send-------4

5-----------clear   to send------>5

OS1 PWR  6-----------data set ready------->6

7-----------signal ground---------7

8--received line signal detected--8   (aka Carrier Detect)

20<----------data terminal ready---20  PTR POWER ON

FIGURE 2      Osborne 1 cabled to NEC 3510 Spinwriter (DTE)

(Same as Figure 1)

Osborne 1 (DCE)                  NEC 3510 Spinwriter (DTE)

PIN                               PIN

(terms relative to DTE)

1----------frame ground-----------1

2<----------transmit data---------2

3-----------receive  data-------->3

4<----------request to send-------4

5-----------clear   to send------>5

OS1 PWR  6-----------data set ready------->6

7-----------signal ground---------7

8--received line signal detected--8

20<----------data terminal ready---20  PTR POWER ON

FIGURE 3      Eagle Spirit connected to Centronics 704 (DTE)

(Full function Null modem cable)

Eagle Spirit (DTE)              Centronics 704 (DTE)

PIN                               PIN

NOTE-->     (terms relative to Centronics 704)

1----------frame ground-----------1

2--\/-------transmit data---------2 (2 & 3 reversed in cable)

3--/\-------receive  data---------3

4--\/-------request to send-------4 (4 & 5 reversed in cable)

5--/\-------clear   to send-------5

6--\/-------data set ready--------6 (6 & 20 reversed in cable)

7-----------signal ground---------7

8--received line signal detected--8

EGL PWR 20--/\-------data terminal ready---20  PTR POWER ON

Note: "reversed" means that pin 2 on the Eagle-side connector

has been re-wired to be pin 3 on the printer-side

connector (and the same for pin 4 and 5, pin 6 and 20).

FIGURE 4      Osborne 1 cabled to Strobe Plotter (DTE)

(Same as Figure 1)

Osborne 1 (DCE)                  Strobe PC100 (DTE)

PIN                               PIN

(terms relative to Strobe PC100)

1----------frame ground-----------1

2<----------transmit data---------2

3-----------receive  data-------->3

4<----------request to send-------4

5-----------clear   to send------>5

OS1 PWR  6-----------data set ready------->6

7-----------signal ground---------7

8--received line signal detected--8

20<----------data terminal ready---20  PLTR POWER ON

FIGURE 5      Osborne 1 (DCE) cabled to Smart-modem (DCE)

(simplest possible null modem cable)

Osborne 1 (DCE)                  Smart-modem (DCE)

PIN                               PIN

(terms relative to Osborne 1)

2<----------receive  data---\/----2 (2 & 3 reversed)

3-----------transmit data---/\--->3

7-----------signal ground---------7

Note: the Smart-modem assumes that it neither device will

transmit fastest than the other device can receive,

thus the cable has no wire connection for one device

to signal the other device to pause in transmitting.

Also, with no signal for "device ready" (normally pins

6 and 20), both devices are assumed to be powered up

and ready when attempts are made for use.

FIGURE 6      Osborne 1 cabled to Houston Instruments

Plotter (DTE)

Osborne 1 (DCE)                  DMP-7 (DTE)

PIN                               PIN

(terms relative to plotter)

1----------frame ground-----------1

2<----------transmit data---------2

3-----------receive  data-------->3

4<----------request to send-------4 (4 & 5 are tied together

5-----------clear   to send------>5  in the plotter)

OS1 PWR  6-----------data set ready------->xx (wired to pin xx to select

7-----------signal ground---------7   baud rate)

20<----------data terminal ready---20  PLTR POWER ON

Note: pin 6 from DCE (host equipment) is wired to one

of several pins to specifiy baud rate. For an

Osborne1, pin 6 is wired to pin 17 to specify 1200 baud.

Trade-marks: the following are trade-marks belonging to their

respective vendors: Centronics 704, Eagle Spirit XL, Houston

Instruments DMP-7, IBM, NEC Spinwriter 3510, Osborne 1,

Smart-modem, Strobe PC100

########################################################################################################