Batteries for the PX-8

Roger P. McConochie

1-313-995-3452

The single item which every PX user will eventually

require is a replacement battery pack.  In meeting this

need, users have a choice among at least three options.

(1)  First, a user might choose to purchase the

genuine Epson item.  These battery packs (Epson RB 201,

#Y203-503-000), still being imported from Japan by Epson

America, are available from Computer Laboratories, in

Minnesota, at 1-800-888-9062.  The price is about $70.

They also offer a full range of PX (and QX-10/16) repairs.

(2)  The second option is to purchase an "Epson-

equivalent" battery pack.  These are offered for $35

(plus S & H) from SnyderScope International, 111 South

Marshall, Suite 202, El Cajon, CA 92020 (1-619-442-2299).

For their eight-page catalog of QX and PX items (computers,

peripherals, accessories, parts, software, and supplies),

users are invited to send a self-addressed, double-stamped

($.45) envelope.  They present themselves as a small-scale,

friendly, informal organization: users do well to expect to

wait a month to receive the catalog and another month to

receive items which have been ordered.

(3)  A third alternative is for users to have

replacement battery packs assembled by a local hobby shop

(or, for users handy with a soldering iron, to build their

own), at a cost of $13-35.

The heart of such a pack is a set of four "sub-C"

size, standard-charge, tabbed, 1.2V 1100mAh nicad

batteries.  Hobby shops sell the Sanyo N-1200-SC cells for

about $6 each, whereas Radio Shack offers an equivalent

battery for $5.99 per package of two (#23-190).  Note

that, although "fast-charge" nicads (such as the Sanyo

"N-1200-SCR" cells) are available, they offer little

advantage.  While they might be more durable, they could

not be charged more quickly than the standard batteries:

although the PX adapter is rated at 600 mAh, a resistor

within the PX prevents the batteries from being charged

faster than the standard-charge rate for the Epson nicads

(150-200 mAh, with a 330 mAh maximum).

Following the model provided by the Epson battery

pack, the new unit should be assembled by soldering

together (in series) the four batteries.  At the lower end,

the connection should be made using insulated wire (rather

than the uninsulated tabs or a section of bare wire), to

avoid having the batteries short out when jammed into the

battery compartment of the PX.  Because replacement packs

(with soldered connections) are likely to be slightly

longer than the Epson units (with their welded joints),

users may care to create another 1/8th-inch of space in the

battery compartment by removing the plastic piece at the

end of the compartment, using a utility knife to smooth out

the rough edges which remain.

Because the battery connector is Epson-specific,

the user may care to cut the Epson leads in half, adding

a standard connector (with the female connector on the

battery leads, to minimize the chance of shorting out the

battery).  Note that only the smallest of connectors will

fit into the battery compartment of the PX-8.  The user

might choose a "dean pin" or, at a slightly greater cost,

a pin with distinct positive and negative leads, so that--

as with the Epson connector--it would be impossible to plug

in the battery incorrectly.  Following this procedure, new

battery packs could be made, using leads and the standard

connector, which could be swapped with other battery packs

as necessary.  The pack could then be finished off by

taping the batteries together, taping over both ends of the

pack, and then enclosing the four batteries in new "shrink

wrap."

#################################################################################################################################################################################################################################################################################################################################