Making BIOS Calls from BASIC on the Geneva

Many people are aware that you can use BIOS operations to give

BASIC programs some of the power and speed that was formerly

limited to machine language programs.  Few, however, are aware of

the techniques required to do this.  In the example program

below, you will see how a BIOS routine (ADCVRT) can make a BASIC

program more powerful and read the internal and external

voltages.

Learning how to use BIOS calls can be very useful for new

programmers.  It allows them the ease and simplicity of writing

in BASIC and then reverting to the more difficult machine code

only when specialty problems of speed or power arise.  The end

result can be powerful programs written with ease.  Some knowledge

of machine code and operating systems is required.

General Procedure

You begin by writing your program in BASIC, leaving the machine

portion undone.  When you are satisfied that all the BASIC

keyboard I/O routines, menus, etc. are in working order, you can

develop the machine code portions that will give you the

additional power.

Developing the Machine Language Portion

Here is the sequence of tasks the machine language portion must

accomplish to handle a BIOS call and pass the results back to

BASIC:

1.  Discover the warm boot address.

2.  Add a certain number to it to call the particular BIOS

routine.  Each BIOS routine you call requires a different

number to be added to it to address that routine.

3.  Store this updated address to memory.  The stored address

(which is the sum of the WBOOT address and an added offset)

equals the calling address of the desired BIOS routine.

4.  Call the desired BIOS routine (in the example below it reads

voltage through the A/D port).

5.  Return to the BASIC program where the data retrieved from

the called routine is converted to meaningful numbers to be

displayed on the screen.

The machine language program that does this is shown below.

Once the machine language program is developed, a couple more

steps remain.  The machine language op codes must be be converted

to decimal codes.  Finally, the converted machine language (now

in decimal format) must be inserted into upper memory (usually

the USER BIOS area) with POKE statements assisted by READ/DATA

statements.

If you choose the USER BIOS area to write your machine code

portion in, you must set aside enough room with the CONFIG

program.  Since the machine program is less than one page of 256

bytes in the example program, the USER BIOS is located at hex

address EB00.  If the routine were more than one page, the

starting address would move down one page for each page of code.

For example, a two page routine would have to start at hex

address EA00.

WARNING: Do not attempt to measure A.C. voltages of any

magnitude or D.C. voltages greater than 2 volts.  You can

seriously damage the Geneva if you try to do this.

Sample Program that Reads the Geneva Battery and External

Voltages

10 REM This is a program to read internal battery voltages and

external D.C. voltages up to 2 volts.

20 REM It is written in BASIC, but it calls machine language

routines in the USER BIOS.

30 CLS

40 IF PEEK(&HF00B)<>1 THEN PRINT "USER BIOS SIZE IS NOT 1 BLOCK--

USE CONFIG PROGRAM TO CORRECT THIS: SET TO 1 PAGE.":END

50 INPUT "DO YOU WANT TO READ INTERNAL BATTERY OR EXTERNAL

VOLTAGES(B/E)";V$

60 IF V$="E" THEN GOTO 300

65 IF V$="B" THEN 90

70 PRINT "USE CAPITALS -- ENTER B OR E"

80 GOTO 50

90 REM *** SECTION TO DEFINE INITIAL VALUES ***

100 DEFINT A-J,L-Y

110 K=5.7/63             'DIVIDE 5.7 V INTO 64 PARTS FOR INTERNAL

120 Z=2/64

130 ADRS=&HEB00                         'USER BIOS AREA

140 T=0

150 REM *** SECTION TO POKE MACHINE CODE DATA TO DISK ***:

160 FOR I=&HEB00 TO &HEB17:           'THIS IS THE USER BIOS AREA

170 READ A

180 T=T+A

190 POKE I,A

200 NEXT I

210 IF T<>2362 THEN PRINT "ERROR IN DATA LINES!!!!":END

220 IF V$="E" THEN POKE &HEB0B,0

230 REM *** SECTION TO CALL ROUTINE AND CONVERT DATA ***

240 CALL ADRS

250 X=PEEK(&HEB17)      'GET THE STORED NUMBER THAT = VOLTAGE NUM

260 PRINT "NUMBER RETURNED FROM A TO D PORT=" X

270 IF V$="E" THEN PRINT X*Z "VOLTS" ELSE PRINT X*K "VOLTS"

280 END

290 REM EXTERNAL VOLTAGE SECTION

300 CLS:PRINT "NEVER EXCEED 2 VOLTS - USE D.C. VOLTAGES ONLY****"

310 PRINT "ACCURACY LIMITED TO 6 BITS (=1 PART IN 64)"

320 PRINT:PRINT "HIT ANY KEY WHEN READY TO PROCEED"

330 INPUT Z$

340 GOTO 90

350 DATA 42,1,0,125,198,111,111,34,13,235,14,3

360 DATA 205,114,231,31,31,230,63,50,23,235,201,61

Detailed Look at the Machine Code Portion

EB00 LHLD      0001      2A,01,00            42,1,0

EB03 MOV       A,L       7D                  125

EB04 ADI       6F        C6,6F               198,111

EB06 MOV       L,A       6F                  111

EB07 SHLD      EB0D      22,0D,EB            34,13,235

EB0A MVI       C,03      0E,03               14,3,

EB0C CALL      EA72      CD,72,EA            205,114,231

EB0F RAR                 1F                  31

EB10 RAR                 1F                  31

EB11 ANI       3F        E6,3F               230,63

EB13 STA       EB17      32,17,EB            50,23,235

EBI6 RET                 C9                  201

The Geneva will read voltages internal and external if the ADCVRT

BIOS function is called.  To call it, you must know the address

where this routine resides.  The address is equal to the warm

boot address plus the hex number 6F.  So the operation of reading

voltages takes place in these steps.

In all CP/M machines, the warm boot address can always be located

by reading the data stored in locations 1 and 2.  A 16-bit read

instruction will read both locations at once, and store the warm

boot location in the HL register if this instruction is:

LHLD 0001

The LHLD instruction causes the 16-bit read that reads both

addresses.  The 0001 gives the starting address 1.  The WBOOT

address has now been read from locations 1 and 2.

Next, addition cannot be done in the HL register, so the data

stored there must be moved from the HL (called L below) to the

accumulator (called A below).  All addition is done in the

accumulator.  This move operation is done with the instruction:

MOV A,L

Now that the warm boot address has been moved to the accumulator,

the next step is to add hex number 6F to it with this

instruction:

ADI 6F

The first stage is now done.  The number in the accumulator

represents the address of the BIOS routine that calls the ADCVRT

routine.  We must store this number for future use.  This magic

number is stored back to the HL register with the command:

MOV L,A

Finally, the number must be stored one more time in a permanent

memory location so it can be called when needed.  It is done in

this program with the command:

SHLD EB0D

Now that the address to call has been determined and stored at

the address directly after the CALL instruction, the next step is

to determine whether the voltage read will be an internal

voltage (battery voltage) or an external voltage.  To make this

choice, a certain number must be placed in the C register just

prior to calling the voltage reading routine.  In the example

below, the internal battery voltage is read by placing a 03

into the C register with the instruction:

MVI C,

At this point all the preliminary work has been done and it is

time to call the routine that will read the voltage.  Calling

this routine will leave a number in the accumulator that

represents the voltage read.  This number will be from 0 to 63.

If you place a 03 in the register before calling the routine,

you read battery voltage.  Each of the 64 numbers in the

accumulator represents the battery voltages from 0 to 5.7 in 64

steps.

If you had placed a 0 in the C register before calling the

routine, each of the 64 numbers you find in the accumulator

represent external voltages from 0-2 V.D.C. in 64 steps.

At this stage all preliminary work has been done.  All you need

to do now is call the routine.  This is done with the command:

CALL EA72

NOTE: The EA72 is a dummy address; it is replaced by the data

provided by the SHLD instruction above.

The Voltage has been read by calling the ADCVRT routine, but the

number stored is an 8-bit value and the port is only capable of

reading with 6 bits of accuracy.  So only 6 of the 8 bits read

have useful information.  To eliminate the right two useless

bits, you issue rotate right commands like this:

RAR

RAR

Masking is performed to get rid of any useless carry bits that

may have crept in from the left.  This is done with the command:

ANI 3F

The final results are now stored at a safe location waiting for

BASIC to read and convert it.  This is done with the command:

STA EB17

You have now found the location, called the routine, read the

data, removed the useless portions and stored the results.  The

final step is to return to BASIC.  This is done with the command:

RET

By multiplying a compensation factor by the number you stored,

BASIC can now read voltages with an accuracy of 1 part in 64.

Notes on Using the ASM Program

The machine program shown must be processed by the ASM program to

generate the hex file that will be used.  Before the ASM program

can use the machine file, certain alterations must be performed.

First, a starting address is needed for the ORG statement.  Since

you are using a one-page USER BIOS, the program is ORGed

(started) at EB00.  Next, the assembler requires that all hex

numbers end with the letter H.  An example of how this looks when

completed is shown below.

ORG       0EB00H

LHLD      0001

MOV       A,L

ADI       6FH

MOV       L,A

SHLD      0EB03H

MVI       C,03H

CALL      0EA72H

RAR

RAR

ANI       03FH

STA       0EB17H

RET

Using the HEX file

When the ASM program has processed your source machine code file,

it produces two new files: one with a .HEX extension, and one

with a .PRN extension.  It is the .HEX one that you need to get

the HEX codes necessary to convert into your BASIC program.  The

HEX file looks like the one shown below:

:10EB00002A01007DC66F6F2203EB0E03CD72EA1F50

:07EB10001FE63F3217EBC9BD

:0000000000

There is definite order to this seeming madness.  As you can see

by looking at the the organized version below, the first portion

is starting line numbers and addresses.  This is followed by the

actual data.  The last part of each line is a checksum digit to

insure accuracy.  The file is terminated by a line of zeros

instead of the usual control Z.  The only part that is useful is

the 16 pairs of numbers on each line (separated by parentheses in

the example below).  These are the number pairs that are

converted to decimal and POKEd into upper memory to form the

machine language part of the program.

:10  EB00 00   (2A 01 00 7D C6 6F 6F 22 03 EB 0E 03 CD 72 EA 1F) 50

:07  EB10 00   (1F E6 3F 32 17 EB C9) BD

:0000000000

The Hex pairs of numbers can be converted in BASIC by using the

PRINT command with the &H prefix.  For example, to convert the

first two pairs on hex numbers above you would enter:

PRINT &H2A

PRINT &H01

The computer would return these values (in BASIC):

42

1

By repeating this sequence, all of the hex data pairs can be

converted to decimal numbers to be poked into the upper memory or

the USER BIOS area.

There is a simpler way to directly insert the hex data into upper

memory.  The DDT program will automatically load the file into

upper memory if the file has been given a starting address with

the ORG command.  This is done by entering a statement like this:

DDT FILE.HEX

#######################################################################################