ADDRESSES FOR GENEVA INTERRUPT ROUTINES

by Justin Bell

Routines  to  handle interrupts for the Geneva should  always  be

written above 8000 hex as one user recently found out.   He wrote

a  routine to handle the interrupt generated by the free  running

counter and located this routine in the 1st page of the TPA area.

He  had checked his routine again and again for correctness,  but

the  routine  would  always blow up.   Baffled,  he  called  Tech

Support for help.

The problem was caused by the bank swiching nature of the Geneva.

The  1st  32K (0 to $7FFF) of the   Geneva's memory exists in two

different  places.   There is a 32K bank of RAM and a 32K bank of

ROM.   During  normal  program operation,  the 32K  bank  of  ram

memory is active.   However, the Geneva swithces over to the ROM

bank in these three cases:

1. During initial boot up.

2. Whenever BIOS or BDOS calls are being executed.

3. Whenever an interrupt is generated.

Since  the  user was calling his routine when  an  interrupt  was

generated  by  the  free running counter,  the Geneva  would  not

execute  the  users  routine he had so neatly written  into  RAM.

Instead  the Geneva would bank switch over to the ROM  bank  and

try  to execute who-knows-what code that was in the same  address

of the ROM bank.  The obvious result was binary bafflement.

Moral:  Write routines for interrupt handling above 8000 hex.  An

excellent place is the user BIOS area which always ends a address

EBFF  hex.   See  the technical note below to see where the  USER

BIOS starts.

See  appendix  C  of  the SYSTEMS ESSENTIAL manual  for  a  brief

explanation of how to use interrupts with the function keys.

TECHNICAL NOTE:

The  Starting address of the USER BIOS area varies with the  size

set with the CONFIG program.  It starts at EB00 when the size  is

one page but, the starting address moves downward as the size  is

increased.  Example:

# of pages         start address       end address

1                   $EB00               $EBFF

2                   $EA00               $EBFF

3                   $E900               $EBFF

4                   $E800               $EBFF

5                   $E700               $EBFF

##########################################################################################