The available info on cabling for the Geneva can be *very* confusing!

The descriptions often conflict with each other, and even the dealers

sometimes don't know what they've got.

Here is a quick rundown as to who's on first, etc.  Where I refer to

a cable "... terminating in an XYZ connector ...", that refers to the

peripheral end - all the cables have the 8-pin mini-DIN on the computer

end, of course!  Note, also, that fairly complete info (including pinouts)

is available in the Geneva's Operations Manual.

**********

#723 - terminates in a standard-size 6-pin DIN.  This cable is used to

interface to the P-80, P-80X and P-40 portable printers, as well as the

TF-20 disk drive (a dual 5.25" floppy drive system, AC-powered).

#724 - terminates in a DB-25 connector, and is colored light gray.

This is a "straight-thru" RS-232 cable, which is used with external

modems (e.g. the CX-20, or Hayes Smartmodem) or any device that follows

DCE ("data set", or modem) type pinouts.

#725 - terminates in a DB-25 as well, but is colored a very dark gray.

This cable is a "null modem" configuration, where certain lines to the

other device are "crossed over" in order to provide the proper

handshaking.  It is used with serial printers, and to hook up to other

computers (e.g. a QX-10 or QX-16, and others) that have a standard DTE

("data terminal") pinout.

#726 - terminates in a mini-DIN on both ends.  This cable is used

primarily for the PF-10 portable 3.5" disk drive, but it can also be

used to hook two Genevas together by their RS-232 or serial ports, for

the purpose of direct communication and data transfer.

**********

Hope this is of some help ...

:: Mike ::

76703,4051

###############################################################