HOW TO PRINT PORTABLE WORDSTAR FILES TO UNSUPPORTED

PRINTERS WITH THE GENEVA PX/8

Portable WordStar is configured for an Epson RX or FX

printer.  When printing Portable WordStar files to a printer

other than an Epson RX or FX, the printout may not be

formatted as desired because of conflicting or unrecognized

printer codes used by WordStar.  The file may be printed with

incorrect line spacing, some unwanted characters may be

printed, or it may be printed in a typestyle other than one

you prefer.  These problems can be overcome by PIPing the

file to the list device.  The following is a description of

this procedure:

After you have created a file, store it as you normally

would and go back to the "Opening Menu".  Select "P" for

PRINT a file.  Provide the name of the file to print when

prompted.  WordStar will then display:

DRIVE FILE OUTPUT (Y/N): <enter Y for Yes>

OUTPUT FILE NAME? <supply a file name>

<answer the other print questions accordingly>

This will create a file on disk formatted for

printing.  Exit WordStar to CP/M.  At the prompt, enter

"PIP LST:=Filename[Z]" (where Filename is the name of the

output file created from the WordStar print routine).  Enter

a Z in square brackets after the file name, so PIP will strip

the eighth bit.  The [Z] may not be required on all printers,

but if you were not to use it on an Epson printer, the last

character in each word would be printed in italics.

SENDING CONTROL CODES TO AN UNSUPPORTED PRINTER

FROM PORTABLE WORDSTAR

Using a procedure similar to the one described above, you can

send certain control codes to your printer using Portable

WordStar.  If you create a file in WordStar, you can embed an

Escape code by typing CTRL P, then ESC, then the code.  For

example, on the Epson LQ-1500 or an Epson printer with an NLQ

board, to turn on Letter quality, you would enter

CTRL P ESC x 1  (no spaces between keystrokes).

This will get embeded in the file, but will not print in a

file that has been formatted for printing through the

WordStar print routine.  You can print a file with an embeded

Escape code if the file is saved (rather than printed) to

disk, but it will not be properly formatted.  If you need an

Escape code to access a particular feature of your printer

you could create a file that consists only of the Escape

code, store it to disk, PIP it to the printer, then PIP your

formatted file to the printer.  The following is an example

of how to print a Portable WordStar file to an NLQ Board in

Letter Quality.  (In this example, the files are stored on

drive A and PIP is in drive C.)

Create a file called "A:LQON.DOC" in WordStar.  In that file,

enter CTRL P ESC x 1 (type a "P" while holding the Control

key, press the ESC key, type an "x", then a "1" without

spacing between characters).  Save this file.  Create a file

that will contain the text you want to print and call it

"A:TEXT.DOC".  When you have finished that file, "Print" it

to disk using the output file name "A:TEXT.PRT" (the

extention ".PRT" is recommended so you know that this file

has been formatted for printing).  Exit WordStar to CP/M.

Enter the following at the prompt:

C:PIP LST:=A:LQON.DOC

wait for the prompt, then type:

C:PIP LST:=A:TEXT.PRT[Z]

You can select a different color on the JX-80 using this

procedure.  For example, for printing in Orange, you would

create a WordStar file with CTRL P ESC r 5, then PIP it to

the printer as described above.

The idea for PIPing WordStar files to the list device is from

THE EPSON CONNECTION:  APPLE by Darnall and Corner.

Richard Rockwell, Epson Mid Atlantic, November 1984

eof.

#######################################################