MCTUTY: MICROCASSETTE TAPE UTILITY PROGRAM VERSION 0.8

This paper covers the functions of the MCTUTY (MicroCassette

Tape  UTIlity) Program,  version 0.8,  for the Epson PX-8 & HX-40

computers.     The program provides 5 basic utility functions for

use with microcassettes:

1) VERIFY:

Test all the blocks and count the number of errors found.

2) DIRECTORY:

Recover and correct a tape directory that was garbaged.

3) CONDENSE:

Move  files stored at the back of a tape to the front  of  a

tape where all the open areas are.

4) DUMP:

Display data stored on a tape in HEX format.

5) RECOVER:

Read data from a tape and copy it to another storage device.

Any "bad" blocks are skipped over and the copying continues.

*** PLEASE NOTE ***

Practice  using this program using a "junk" tape with  dummy

files BEFORE you try it on the real thing.

NEVER run this program without the AC Adapter CONNECTED.

Play  it  safe,  ALWAYS copy all your files in RAM  Disk  to

Floppy  Disk  or Tape and remember to use the system function  to

REMOVE said backup tape.

STARTING THE PROGRAM:

Before  running the program,  enter the system menu and make

sure that the microcassette is not mounted.   Once done, exit the

system screen and select MCTUTY through the main MENU program  or

by typing "MCTUTY<RETURN>" at the CP/M level.

There are 2 keys that have special functions: STOP & ESC.

STOP  will  return the program to the main menu;  however if  the

program is in the main menu, STOP will cause the program to exit.

ESC will return the program to the previous menu.

These keys may be used at any time to move backward from the

current menu to an earlier menu.  Also, at any point in any menu,

CTRL-P may be used to obtain a hard copy of what you're doing.

Once  in the main menu,  the number 1 - 5 may be entered  to

select a given function.  Pressing the RETURN Key is not required

to  select  a  function.   Use  of each function  is  covered  as

follows:

(1)  VERIFY:

Verify is used to test data stored on a tape.  VERIFY allows

for a selection of:

1    Single File (Selectable)

2    All Files (Until CTRL-STOP is pressed)

Just press the number of the selection needed.

If  "1" is pressed,  the program will read the directory  at

the  start  of the tape and display a menu of files that  may  be

verified.   Just  type  the number of the file that needs  to  be

tested  and then press the RETURN Key.   If more that one file is

to  be verified,  more than one number may be entered like  this,

"1,3,5,7".

If "2" is pressed, the program will wind to the start of the

tape  and  test every file it finds until CTRL-STOP  is  pressed.

This  function does NOT stop running the cassette when the end of

the tape is reached.

In both cases the information displayed is as follows:

FileName.Ext    HDR    00000    EOF Error=00000

^               ^      ^        ^         ^

|               |      |        |         |

file name       header current  End       Total

of file         was    block    Of        Errors

found  number   File      Found

(2)  DIRECTORY:

This  function is designed to restore a tape directory  that

was  garbaged  for some reason.   The recreation of  the  correct

directory may be done from the beginning or after a selected file

to the end of the tape or wherever you wish to stop.

1    From Some File

2    All Files

If  "1" is entered,  the program will read the directory  at

the start of the tape,  then display a list a files that would be

the  starting point.   The starting point would be after the file

selected.   From  that  point on,  the program  will  update  the

directory based on what is found on tape.   Use CTRL-STOP to tell

the program that there are no more files to be found.

If  "2" is entered,  the program will search for files  from

the  start  of the tape.   The function of the search is like  #1

above.   Use CTRL-STOP to tell the program that there are no more

files to be found.

After  the search for files is finished,  the user is  given

the  option  of renaming any of the file  names.   This  function

exists because the same file name may exist on the tape more than

once.  In order to rename a file, just enter:

number=filename.ext<RETURN>

1=MYFILE.DOC

A few words of warning about known bugs in this function:

The record count of a recovered file will be 4 or 5  records

higher  than  what  it really is.   While this  doesn't  cause  a

problem  for text files that mark the end of file with a  CTRL-Z,

reading other files may be a problem,  if you try to pull a  copy

of  the  file  from tape.   The RECOVER Function can be  used  to

correct this problem and read the file correctly.

This  function  does  not  seem to test  for  end  of  tape.

Therefore,  you should watch for the end of a tape or the program

will cause the tape drive to keep running and running EVEN AT THE

END OF A TAPE.

The  RENAME part of the DIRECTORY Function does NOT  convert

lower case to upper case.   Thus, you could create a file name in

lower case that can NOT be recovered.  ALWAYS enter file names in

upper case.

(3)  CONDENSE:

If  you  must use this function,  please use this  one  with

care.   It  is possible to kill all the files on the tape and not

have  a way to recover any of them.   Once the red  record  light

comes on (or on then off),  DO NOT STOP this function or you will

lose your files.   Once started,  the function could take as long

as 45 minutes to do one side of a 30 minute tape.

(4)  DUMP:

The  DUMP  function allows a user to examine  the  data  and

header  information stored onto a tape.   This function allows  2

options:

1    Single File (Selectable)

2    All Files

If  "1" is entered,  the tape directory will be read and the

file or files to be dumped are selected by number.  More than one

file is selected like this,  "2,4,6,8".   The program will return

to the file selection menu when finished dumping out the file(s).

If  "2" is selected,  the program will wind the tape to  the

beginning and dump out what's stored from that point on.

There are 2 known problems with this function:

Like the RECOVER function,  this function does NOT test  for

the end of a tape.   Thus,  unless stopped by CTRL-STOP,  the the

program would keep the tape drive running on and on,  even at the

end of a tape.

The  normal CTRL-S (PAUSE) does NOT halt a display  listing.

If  you  want to look over all the data,  use CTRL-P to send  the

data to the printer.

The format of the data is as follows:

(All numbers are in HEX)

Block beginning tape count = HH HH <--- this is the value of

the tape counter at

the start of the

block.

Block discrimination field = H1 H2 H3 H4

H1 = The type of block:  48H = Header Block

44H = Data Block

45H = EOF Block

H2 H3 = The Block Number (0000H - FFFFH)

H4 = Block Copy Number:  00H = First Copy

01H = Second Copy

This information is followed by 256 hex bytes of data.

(5)  RECOVER:

This function copies files from the microcassette to another

storage device.   Any errors, such as bad blocks, are ignored and

256 bytes of spaces are returned for this type of error.  Errors,

such  as the wrong number of records in the header,  are  ignored

and  only the correct number of legal records are  copied.   This

function offers 2 options to recover:

1    Single File

2    All Files

If "1" is selected, the tape directory is read and a menu of

possible files to be recovered is displayed.  A file is  selected

by  typing the number and a return.   The program will ask  where

the  file is to be copied to.   You can NOT copy a file from  the

microcassette  to the microcassette,  please do not select  drive

"H:"  as the destination drive.   Other files may be  copied,  by

waiting  for  this copy process to finish and entering in  a  new

number.

If "2" is selected,  every block on the tape will be  copied

into  one  master  file.   Like option #1 above,  the  drive  and

filename  of the file to copy to is selected by the  user.   Also

drive "H:" can't be used as the destination drive.   I'm not sure

why this option exists,  because all the blocks on a  tape,  even

the  directory,  are  stored  on one large  unuseable  file.   In

theory,  one could use DUMP or DDT to look at all the blocks, but

users  will have problems understanding where one file  ends  and

the other begins.

There are a few known bugs with this function:

The file name selection does not convert lower case input to

upper  case.   Therefore,  it is possible to enter a output  file

name in lower case and CP/M would not be able to read it.  Always

enter all file names in upper case.

When  option #2 is selected,  the program does NOT test  for

the end of a tape.   Thus, unless you enter CTRL-STOP to tell the

program  to stop copying,  the program will keep running the tape

drive even after the end of the tape is reached.

DISCLAIMER:

This software is given out AS IS and Epson shall not be held

liable for any damages,  direct or indirect,  caused through  the

use  of this program or the information contained in this  paper.

Because  the software is given out as is to end users,  Epson  is

not  required  in any way to support or provide updates  to  this

software.

After testing the program for a few days,  it is clear to me

that  there  are several bugs within the  program.   While  every

effort has been made to find as many bugs as possible, it is very

likely  that  there are other unfound bugs within this  software.

This  program  was written by Epson Japan and given out by  Epson

America  to end users for free.   This program may be  copied  by

anyone  PROVIDED  that  the software is copied  on  a  NON-PROFIT

bases.   Epson  America  does  NOT have the source code  to  this

program.

#e is copied  on  a  NON-PROFIT

ba