**>> DO.DOC <<**

This DO file offers two things.

1>> It is useful in and of itself.  It has three utilities for

the Epson PX-8 squeezed into less than 1k, the size of the

allocation group, and therefore the size of the minimum disk

file.

a. There is BAT.COM, a battery program, which reads the

voltage of the PX-8's battery.  BAT.COM lets you keep track of

how much more computing you have left in the Epson.

b. There is a version of David Kozinn's PFK.COM, which

stores and installs a set of assignments for the PX-8's function

keys. This allows you to overcome the tendency of WordStar to

reassign your function keys when you go back into CP/M. But more

importantly, it allows you to reassign your own keys with

abandon, knowing that in a blink you can get back to your

standard configuration.

c. There is a new program, TD.COM, which I wrote to access

the time and date capability of the PX-8. The Geneva with TD is

the world's largest, heaviest digital watch. Perhaps it is the

world's first digital grandfather clock? Never mind. In addition

TD will also write a FILE with the current time in it, and

therefore allow you to use your computer to date text, or as a

recording stopwatch, which can store times accurate to the

second. TD.ASM is available separately in this DL.

Recall that this all resides in 1k, and costs you no more disk

space than any of the programs alone.

2>>  That is the second point of DO.COM.  It illustrates an easy

way of "packing" programs in order to use disk and directory

space very efficiently when both are at a premium, as they

certainly are in the Epson PX-8.  It is slightly remarkable that

DO.COM actually puts four programs (counting DO itself) in 3/4 of

a kbyte.  As you can see by reading the description below, this

still leaves space for a future fifth program, bigger than any of

the first four, without exceeding 1k.

The code which maintains the programs is included in this file as

DO.ASM.  Making a DO file for your favorite set of programs

requires a familiarity with DDT and some knowledge of assembly

language programming, though not much.  You must also have

LOAD.COM and ASM.COM (or their public domain equivalents).  These

programs usually come with CP/M, but not unfortunately, in the

case of the non-disk PX-8.  Equivalents are easy to find.  In

return for this required diddling with DO, you can take any

program, whether you have the source code or no, (I have only

BAT.COM, for example) and make it a part of a compact DO package.

**>> Using DO <<**

B>do

B = BAT.COM

**>>DO  P = PFK.COM

T = TD.COM

Choice:

If your Choice: is B, you get a self-explanatory message and

graphic about the battery voltage.  Very useful; thanks to Bob

Diaz, who wrote this program.

If your Choice: is P, your function keys will be assigned as

follows:

1)dir

2)type

3)c:stat

4)c:pip

5)c:config^M

6)^QF ^N^N cr B cr ^F

This takes you to the top of the WordStar paragraph you're

working in. Function 6 again takes you to the top of the next

paragraph, and so on.  This assumes single spacing, with two

returns at the end of paragraphs.

7)QF ^N^N esc bs

Takes you to the bottom of the paragraph, otherwise like 6).

8)^KB sp ^KK bs ^KR N cr ^QK del

Many thanks to Dick Ezzard's "Shades of WordStar" for this one.

Number 8 reads in a notes file (whose file name is "N") from your

disk as a marked block. The notes file can contain the outline

for what you're writing, a list of references, or ideas for

insertion elsewhere in the text. Writers will find so many uses

for this that you simply won't believe it, and it works

wonderfully with the PX-8's ramdisk. The notes file, N, pops in

and out (see function 9) in a flash.

9)^KW N cr ^KY

The N file was marked as a block ( <B>.N.<K> ) when it was read,

and will now vanish back onto your disk, accepting any changes

you have made while you had it onscreen. A simple ^KY (delete

marked block) will send it away, and restore your file, without

recording any changes you made to N.

0)^QR ^KB ^QC ^KK ^KW

Function 0 marks your entire WordStar text as a block, and

initiates a block write to disk. It will stop and ask for the

name of the file to make. Why do this? It enables you to keep a

growing text as a single file, without WordStar's space-gobbling

BAK files. The size of the document you can edit is more than

doubled. I don't feel that I'm giving up much security either,

since many of the disasters avoided by BAK files are not likely

on the battery-powered PX-8.

Of course, these choices reflect the activities (and

peculiarities) of the author, and you may want other default

functions.  Fair enough.  Just get David Kozinn's PFK.ASM from

this SIG, edit, and assemble.

If your Choice: is T, you get the date and time onscreen.  I know

you get this from the PX-8's program menu, but the menu drives me

bananas, and I never use it.  TD gives you the information where

you need it.  The PX-8 encourages you to go and compute from a

hilltop, and TD allows you to keep track of the world from your

hill, if you want to.

More to the point, if you feed TD (Time&Date) a filename, thusly:

A>DO time1<cr>

"t" at the menu        (or  A>do time1<cr>t   all in one string)

then TD will write a text file called time1 with the time

recorded in it, and display the t&d onscreen as well.  The time

recorded is that at the instant of -> Choice: t.

The time file (time1, t2, Chicago, enlightenm...) can be opened

with WordStar, and text can be added to the date, to give a dated

journal.  In the same way, a series of time files with

descriptive names (lap1, lap2,...) is a recording stopwatch's

record of any event.  The files can be read into a WordStar file

by name (using ^KR), and text added, if desired.

But be careful.  TD does NOT add to a preexisting file.  If the

file you name on the command line (time1, t2, Chicago,

enlightenm...) already exists, TD will REPLACE it. If you want to

append a date to a file you already have, write a time file with

a new name, then go into your existing file with WordStar, and

read in the time.

**>> Making a DO group <<**

Here is a text outline of the DO process, a transcript of the

computer screen as this DO file was assembled, and finally the

text of DO.ASM itself.  Reading back and forth between these

should help make the process clear.  Use DO.PRN, which is written

by the assembler (eg, ASM) while it is assembling your file, to

clarify the address arithmetic.

Briefly:

1. Take the .COM files you wish to DO, and determine their real

sizes using DDT (or EDFIL, or PATCH, or DU).  Most .COM files

will end with 00's which serve no function, and one way the DO

file gains space is to set the actual code of one file over the

00's of another to completely fill disk space.  Here's how:

A>ddt pfk.com          << Type: ddt <program>, followed by <cr>.

DDT VERS 2.2           << ddt loads pfk.com into memory, and says

NEXT  PC               << next free address is 0200 hex, meaning

0200 0100              << that pfk.com stretches from 0100 to 01FFh.

-d                     << "d" is the ddt command to display memory.

0100 11 2A 01 0E 09 CD 05 00 2A 01 00 7D C6 6C 6F 22 .*......*..}.lo"

0110 1F 01 0E FF 21 2E 01 0C 79 FE 0A C8 C5 E5 CD 00 ....!...y.......

0120 00 E1 C1 7E 85 3C 6F C3 17 01 1B D3 01 24 04 64 ...~.<o......$.d

0130 69 72 20 05 74 79 70 65 20 07 63 3A 73 74 61 74 ir .type .c:stat

0140 20 06 63 3A 70 69 70 20 09 63 3A 63 6F 6E 66 69  .c:pip .c:confi

0150 67 0D 08 11 46 0E 0E 0D 42 0D 06 06 11 46 0E 0E g...F...B....F..

0160 1B 08 0D 0B 42 20 0B 4B 08 0B 52 4E 0D 11 4B 7F ....B .K..RN..K.

0170 06 0B 57 4E 0D 0B 59 0A 11 52 0B 42 11 43 0B 4B ..WN..Y..R.B.C.K

0180 0B 57 00 00 00 00 00 00 00 00 00 00 00 00 00 00 .W..............

0190 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 ................

01A0 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 ................

01B0 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 ................

|  | |<- contents of memory as hexadecimal bytes ->| |<-memory as   |

hex address                                          |  ASCII text->|

**>> The first 0C0h bytes of program memory, loaded with PFK.COM <<**

PFK.COM occupies addresses 0100 through 0181h (h=hex, d=decimal

number).  In contrast to what DDT said about the first free address,

0182 to 200 hex is filled with 00's, not the program.  These

positions could contain anything (even another program, heh, heh)

and PFK.COM would perform equally well.  So: PFK.COM actually consists

of 82h bytes (130 decimal) = 0181h - 0100h + 1, which must be packed

into DO.COM in order to use the program.

There are very occasional programs which actually use the 00's

as initialized space, and won't work unless you give it to them.

This is about as frequent as shrimps that whistle.  So now you will

recognize this if you see it, and in the meantime, don't worry.

2. Now edit DO.ASM using the real sizes of your .COM files.  You

need to alter the equates at the top of the program so that the

start and ends of your programs make a tight stack beginning at

0104h or so, right after the "jmp start", which points to the

actual DO code, above all programs.

All CP/M programs are constructed to run at 0100h, but none of

these programs (except DO's jmp start) are there. How do they run

when all their internal addresses are wrong and any jmp will be a

jump into the void? The DO.ASM code moves the program chosen from

the menu to the standard position at the bottom of the TPA

(transient program area), then turns over control to it. That's

why you have just readjusted the equates (startp and endp, for

example) of DO.ASM so that your programs will be moved to 0100h

when they are chosen.

You also must alter the code which sorts program choices.  This

is written in a modular fashion, so that paragraphs can be

duplicated, and the names changed, to accomodate new programs.

Using the start and end addresses of the entire program set, make

enough room for all your programs by changing the space directive

between the ###'s in the middle of DO.ASM.

3. Finally, change the menu by adding your new choices to it.

4. Now assemble DO.ASM to get DO.COM.  After assembly DO.COM is

only a shell of code with gobbledy-gook in the space you have

defined for programs, but it should load (A>do<cr> ) at this

point, and display the menu, then take you back to CP/M when you

ask for a menu choice which is not on the list.

5. Now for the trickiest part, though if you can do the first

sentence in 1. above, you can do this too.  You must pack your

programs within DO.COM, arranging that the starting addresses are

just what you wrote down in the equates.  The ends are less

critical, and if you don't need tightest packing, just be careful

that one program doesn't step on the end of another, and that

start - end = more than the whole program.  Here's the dialog

which created the distribution version of DO.COM, as an

illustration:

________________ COMMENT ________________

A>lasm do              << ASM will work, too.  We are at point 4.

LASM3 11/01/84

Pass 1

Pass 2

03FE                   << First free address above DO.

001H use factor        << We used 1 hex percent of symbol space.

131  input lines read

End of assembly

Toon = ^_              << You will see "Warm Boot", TOON12 is my

<< screen dump program, used to record

<< this dialog from the screen.

A>load do              << This is CP/M's LOAD.COM.

FIRST ADDRESS 0100     << LOAD gives program size, 3 ways.

LAST  ADDRESS 03FD

BYTES READ    00AD

RECORDS WRITTEN 06     << Now have DO.COM = 6x128d byte records.

A>ddt do.com           << Load the assembled shell using DDT.

DDT VERS 2.2

NEXT  PC               << We're using 0100-0400h: still 256d bytes

0400 0100  	       << free under 1024d bytes = 1k = 400h bytes.

-ipfk.com              << Load PFK.COM into DO.COM using "i", then "r".

-r0004                 << This is the "offset" from 0100; addr = 0104h

NEXT  PC

0400 0100

-ibat.com              << BAT.COM

-r0088                 << Note "r" address is the offset from 0100h!!

NEXT  PC

0400 0100

-itd.com               << Here we use another method to get

-r400                  << TD.COM loaded in minimum space.  First,

NEXT  PC               << load above everything else.

0600 0100

-m500,5E8,266          << Then move down just TD.COM, leaving

-^C                    << zeros behind.  Now warm boot to get

Toon = ^_              << out.

A>save 3 do.com        << Immediately save 3x256d bytes = 3/4k to

A>                     << preserve the locations in memory you

<< just constructed: 0400h-0100h=3 "pages".

**>> 8080 Assembly Language for DO <<**

Here is the code. Just write it out to its own disk file with

your text editor, and assemble.  Your .ASM file should start

with the first ";" at the left hand margin.  Be aware that if

you change anything which is not to the RIGHT of a ";", you

are donning the greasy coveralls of a programmer.

May the Force attend you.

;                 ** DO.ASM **

;A "DO-file" of executable programs, which may be

;individually brought to 0100h and run, using

;a choice made from a menu.  This scheme

;saves disk space and directory entries, for

;computers which are short on both, like the

;Epson Geneva PX-8.  This version of the program

;contains 3 useful utilities in 1kbyte, which is

;the minimum file size on the Geneva.

;

;Mike Yarus, 2231 16th Street, Boulder, CO 80302

;Compuserve 73145,513		      July, 1985

;

;Equates which show the addresses of the programs

;Assemble DO to get a .COM file with space in the

;middle to accept the desired programs, then

;load .COM files at these addresses w/ DDT, then

;SAVE to get final executable DO.COM file

;

startp	equ	0104h	;start addr, pfk.com

endp	equ	0187h	;end addr for movbyte

startb	equ	0188h	;start addr of bat.com

endb	equ	0265h	;end addr

startt	equ	0266h	;start of td.com

endt	equ	035Bh	;end addr

;

;General equates

;

bdos	equ	0005h	;bdos jump

wrtstr	equ	09h	;write string fn

rdconsl	equ	0Ah	;read console buffer fn

tpastrt	equ	0100h	;tpa start

lf	equ	0Ah

cr	equ	0Dh

tab	equ	09h

;

org	tpastrt	;0100h tpa start

;

jmp	start	;go to DO, above the programs

;

;############# put programs here w/ DDT ############

;

ds	24Fh	;Space for the programs!!!

;

;############# space for programs above ############

;

;go to menu -> get choice of prgm

;

start	mvi	c,wrtstr;write string

lxi	d,menu	;load menu addr

call 	bdos	;put up memu

;

mvi	c,rdconsl	;get prgm choice

lxi	d,buffsiz	;1 char allowed

call 	bdos		;edit command

;

mvi	c,wrtstr	;clear the choice line

lxi	d,crlf

call 	bdos

;

lda	choice+1	;get letter choice back

call 	ucase		;u/c choice

;

;--- a list of the programs, w/ load addresses ---

;

bat	cpi	'B'	;a B?

jnz	pfk	;no, try next choice

lxi	h,startb	;yes, load prgm B

lxi	d,endb+1	;last byte + 1

lxi	b,tpastrt	;load for run

call	movbyte	;ready to move B

jmp	tpastrt	;give control to B

;

;-----   one of these paragraphs per program   -----

;

pfk	cpi	'P'	;a P?

jnz	td	;no, try next one

lxi	h,startp 	;yes, load prgm p

lxi	d,endp+1	;last byte + 1

lxi	b,tpastrt	;load for run

call	movbyte	;ready to move P

jmp	tpastrt	;run it

;

;-----   one of these paragraphs per program   -----

;

td	cpi	'T'	;a T?

rnz		;no, LAST ONE, out of choices

lxi	h,startt	;yes, load prgm t

lxi	d,endt+1	;last byte + 1

lxi	b,tpastrt	;load for run

call	movbyte	;ready to move T

jmp	tpastrt	;run it

;

;   ------------ end program list ------------

;

;relocate the bytes beginning at  hl

;	      bytes end + 1   at  de

;		    to addr   at  bc

;

movbyte	mov	a,m	;get a byte

stax	b	;put byte to bc

inx	b	;increment put addr

inx	h	;increment get addr

;

mov	a,h	;hi nibble, get addr

cmp	d	; = hi nibble, end?

jnz	movbyte	;no, move another byte

;

mov	a,l	;lo nibble, get addr

cmp	e	; = lo nibble, end?

jnz	movbyte	;no, move another byte

;

ret		;done w/ move

;

;--- take char in a, do crude upper case test ---

;

ucase	cpi	60h	;if bigger, lcase

rm		;already upper case

sui	20h	;make upper case

ret

;

buffsiz	db	1	;the prgm buffer

choice	ds	2	;act # char, then choice

crlf	db	cr,lf,'$'

;

menu	db	tab,'B = BAT.COM',lf,cr

db	'**>>DO  P = PFK.COM',lf,cr

db	tab,'T = TD.COM',lf,cr

db	'Choice: $'

;

end             ;DO, did, done

###########################################################iest digital watch. Perhaps it is the

world's first digital grandfather clock? Never mind. In addition

TD will also write a i	60h	;if bigger, lcase

rm		;already upper case

sui	20h	;make upper case

ret

;

buffsiz	db	1	;the prgm buffer

choice	ds	2	;act # char, then choice

crlf	db	cr,lf,'$'

;

menu	db	tab,'B = BAT.COM',lf,cr

db	'**>>DO  P = PFK.COM',lf,cr

db	tab,'T = TD.COM',lf,cr

db	'Choice: $'

;

end             ;DO, did, done

###########################################################��������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������������DO.ASM.  Making a DO file for your favorite set of programs

requires a familiarity with DDT and some knowledge of assembly

l