# Emulator Enhanced Z80 Instructions

- MUL8	Multiply two 8 bit values	2

- MUL16	Multiply two 16 bit values	3

- DIV16	Divide two 16 bit values	4

- DIV32	Divide two 32 bit values	5

- IMUL8	Multiply two 8 bit signed values	6

- IMUL16	Multiply two 16 bit signed values	7

- IDIV16	Divide two 16 bit signed values	8

- IDIV32	Divide two 32 bit signed values	9

- SHELL	PC DOS Shell function	10

- MOUSE	PC Mouse functions	11

- IPORT	Input from PC port	13

- OPORT	Output to PC port	14

- RMEM	Read from PC memory address	15

- WMEM	Write to PC memory address	16

- SSPD	Set CPU speed	17

- GSPD	Get current CPU speed	18

- OPEN	Open PC file	19

- CLOSE	Close PC file	21

- READ	Read record from PC file	22

- WRITE	Write record to PC file	23

- SEEK	Move PC file pointer	24

- ERROR	Build PC error mesage	25

- FINDFIRST	Find first matching directory entry	26

- FINDNEXT	Find next directory entry	28

- LOAD	Load object file from PC directory	29

- CHDIR	Change PC's working directory	30

- GETDIR	Get current working directory	31

- PATHOPEN	Open PC file using default paths & hard disk paths	32

- CLOSEALL	Close single file or all files	34

#### MUL8 D, E

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	20H

Description:

Multiply register D by register E and return result in DE.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

V	Clear if D is zero; set otherwise

C	Clear if D is zero; set otherwise

Example:

100		LD	D,15H	; load D with 15h

110		LD	E,40H	; load E with 40h

120		DEFB	EDH		; multiply 8 bit unsigned values

130		DEFB	20H

After execution DE will contain 540h

#### MUL16 HL, DE

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	21H

Description:

Multiply register HL by register DE and return result in HL:DE.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

V	Clear if HL is zero; set otherwise

C	Clear if HL is zero; set otherwise

Example:

100		LD	HL,2015H	; load HL with 2015h

110		LD	DE,0140H	; load DE with 140h

120		DEFB	EDH		; multiply 16 bit unsigned values

130		DEFB	21H

After execution HL will contain 28h and DE will contain 1A40h

#### DIV16 HL, DE

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	22H

Description:

Divide register HL by register DE and return result in HL and remainder in DE.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

P/V	Set if divide by zero; reset otherwise

C:	Set if divide by zero; reset otherwise

Example:

100		LD	HL,2175H	; load HL with 2175h

110		LD	DE,40H	; load DE with 40h

120		DEFB	EDH		; divide 16 bit unsigned values

130		DEFB	22H

After execution HL will contain 85h and DE will contain 35h

#### DIV32 HL:DE, IX:IY

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	23H

Description:

Divide registers HL:DE by registers IX:IY and return result in HL:DE and remainder in IX:IY.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

P/V	Set if divide by zero; reset otherwise

C:	Set if divide by zero; reset otherwise

Example:

100		LD	DE,2175H	; load HL:DE with 402175h

110		LD	HL,40H

120		LD	IX,4578H	; load IX:IY with 14578h

130		LD	IY,0001H

120		DEFB	EDH		; divide 32 bit unsigned values

130		DEFB	23H

After execution HL:DE will contain 32h and IX:IY will contain 9005h

#### IMUL8 D, E

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	24H

Description:

Perform a signed multiply of register D and register E and return result in DE.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

P/V	Set if DE ≠ sign-extend of E to 16 bits or overflow; reset otherwise

P/V	Set if DE ≠ sign-extend of E to 16 bits or overflow; reset otherwise

Example:

100		LD	D,15H	; load D with 15h

110		LD	E,40H	; load E with 40h

120		DEFB	EDH		; multiply 8 bit signed values

130		DEFB	24H

After execution DE will contain 540h

#### IMUL16 HL, DE

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	25H

Description:

Perform a signed multiply of register HL by register DE and return result in HL:DE.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

P/V	Set if HL:DE ≠ sign-extend of DE to 32 bits or overflow; reset otherwise

P/V	Set if HL:DE ≠ sign-extend of DE to 32 bits or overflow; reset otherwise

Example:

100		LD	HL,2015H	; load HL with 2015h

110		LD	DE,0140H	; load DE with 140h

120		DEFB	EDH		; multiply 16 bit signed values

130		DEFB	25H

After execution HL will contain 28h and DE will contain 1A40h

#### IDIV16 HL, DE

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	26H

Description:

Perform a signed divide of register HL by register DE and return result in HL and remainder in DE.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

P/V	Set if overflow or divide by zero; reset otherwise

C:	Set if overflow or divide by zero; reset otherwise

Example:

100		LD	HL,2175H	; load HL with 2175h

110		LD	DE,40H	; load DE with 40h

120		DEFB	EDH		; divide 16 bit signed values

130		DEFB	26H

After execution HL will contain 85h and DE will contain 35h

#### IDIV32 HL:DE, IX:IY

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	27H

Description:

Divide registers HL:DE by registers IX:IY and return result in HL:DE and remainder in IX:IY.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

P/V	Set if overflow or divide by zero; reset otherwise

C:	Set if overflow or divide by zero; reset otherwise

Example:

100		LD	DE,2175H	; load HL:DE with 402175h

110		LD	HL,40H

120		LD	IX,4578H	; load IX:IY with 14578h

130		LD	IY,0001H

120		DEFB	EDH		; divide 32 bit signed values

130		DEFB	27H

After execution HL:DE will contain 32h and IX:IY will contain 9005h

#### SHELL A, HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	28H

Description:

Shell to DOS and execute the command pointed to by register HL. If the command is null (only a zero terminator) then this function shells to a DOS prompt. If Z flag is set then the command executed ok and register BC contains return code else register A contains the error number.

If register A is non-zero then a push any key to continue message will be displayed when the command finishes before the TRS-80 screen is re-displayed.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if shell command ok; reset otherwise

Example:

100		LD	HL,CMD	; load HL with address of command

110		LD	A,01H	; display push any key to continue message

120		DEFB	EDH		; execute shell command

130		DEFB	28H

.

.

.

200	CMD	DEFM	‘DIR’	; zero terminated command to execute

210		DEFB	00H

#### MOUSE A, BC, DE, HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	29H

Description:

This function allows access to the PC mouse, there are 6 sub-functions.

Function 1:	Get X corr, Y corr and buttons

Before:
	B	= 1

After:
	HL	= mouse cursor X value
	DE	= mouse cursor Y value
	A	= button status
		bit 0 reset if right button pushed
		bit 1 reset if middle button pushed (left & right pushed for 2 button mice)
		bit 2 reset if left button pushed

Function 2:	Set X corr, Y corr and set button status

Before:
	B	= 2
	C	= button status
		bit 0 set or reset right button pushed
		bit 1 set or reset left button pushed

After:
	flags	= Z flag set if successful

Function 3:	Get max X corr, Y corr and sensitivity

Before:
	B	= 3

After:
	HL	= mouse max X corr value
	DE	= mouse max Y corr value
	A	= sensitivity (0 to 3); 3 is most sensitive

Function 4:	Set max X corr, Y corr and sensitivity

Before:
	B	= 4
	HL	= mouse max X corr value
	DE	= mouse max Y corr value
	A	= sensitivity (0 to 3); 3 is most sensitive
After:
	flags	= Z flag set if successful

Function 5:	Get mouse type

Before:
	B	= 5

After:
	A	= mouse type (0=two button, 1=three button)

Function 6:	Get X corr, Y corr and buttons

Before:
	B	= 6

After:
	HL	= mouse cursor X value (0 - 1024)
	DE	= mouse cursor Y value (0 -1024)
	C	= buttons currently pushed
		bit 4 set if right button pushed
		bit 5 set if left button pushed
	B	= buttons pushed since last reset using function #2
		bit 4 set if right button was pushed
		bit 5 set if left button was pushed

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:	set according to function type

Example:

100		LD	B,01H	; load B with get X, Y and buttons command

110		DEFB	EDH		; execute mouse command

120		DEFB	29H

130		.			; HL contains X cursor value

140		.			; DE contains Y cursor value

150		.			; A contains button status

#### IPORT A, (HL)

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	2AH

Description:

Read the PC port pointed to by register HL into register A.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:	none

Example:

100		LD	HL,0201H	; read PC game port

110		DEFB	EDH

120		DEFB	2AH

#### OPORT (HL), A

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	2BH

Description:

Write register A to the PC port pointed to by register HL.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:	none

Example:

100		LD	A,0FFH	; load A with FFh

110		LD	HL,0201H	; write register A to PC game port

120		DEFB	EDH

130		DEFB	2BH

#### RMEM A, (DE:HL)

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	2CH

Description:

Read the PC memory address pointed to by register DE:HL into register A. Register DE contains the PC segment and HL contains the address within the segment.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:	none

Example:

100		LD	DE,0040H	; set segment to 0040h

110		LD	HL,0000H	; set address to 0000h

130		DEFB	EDH		; read address 0040h:0000h into A

130		DEFB	2CH

#### WMEM (DE:HL), A

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	2DH

Description:

Write register A to the PC memory address pointed to by register DE:HL. Register DE contains the PC segment and HL contains the address within the segment.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:	none

Example:

100		LD	A,0FFH	; load A with FFh

110		LD	DE,0040H	; set segment to 0040h

120		LD	HL,0000H	; set address to 0000h

130		DEFB	EDH		; write a to address 0040h:0000h

140		DEFB	2DH

#### SSPD A

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	2EH

Description:

Set CPU speed to value defined by register A.

bits 0-1
00	2/1.77mhz
01	4mhz
10	1mhz
11	8mhz

bit 2	Turbo speed if set (bits 0-1 have no effect if set)

bit 3	Speed port locked if set (prevents normal Z80 programs, like DOS, from resetting speed)

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

None

Example:

100		LD	A,0CH	; Set turbo speed & lock port

110		DEFB	EDH		; execute speed set command

120		DEFB	2EH

#### GSPD A

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	2FH

Description:

Get CPU speed and return value in register A.

bits 0-1
00	2/1.77mhz
01	4mhz
10	1mhz
11	8mhz

bit 2	Turbo speed if set

bit 3	Speed port locked if set

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

None

Example:

100		DEFB	EDH		; Get current CPU speed

110		DEFB	2FH

#### OPEN BC, DE, HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	30H

Description:

Opens the file specified by the path pointed to by HL. The path must conform to PC standards and be zero terminated. The BC register is used to define the type of file access. If the file is being created the DE register is used to define the access permission.

If the open is successful the Z flag is set and the file’s channel is returned in the DE register else the Z flag is reset, DE is loaded with FFFFh and the A register contains the error number.

File Access:

bits 0-1 of BC:
	00	open file read only
	01	open file write only
	10	open file read/write

bit 6 of BC	create file if doesn't exist (create)
	bit 7 of BC	used only with bit 6 set. If file exists, an error is returned
	bit 9 of BC	if file exists then file is truncated (truncate)
	bit 10 of BC	the file pointer will be set to the end of the file prior to each write (append)

File Access Permission:

bit 7 of DE	if set, permission to write to file
	bit 8 of DE	if set, permission to read to file

NOTE:	only has effect when file is being created. To allow read & write access both bits must be set.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file opened ok; reset otherwise

Example:

100		LD	HL,PCFN	; load HL with address of filespec

110		LD	BC,0241H	; create or truncate file & open in write only mode

120		LD	DE,0180H	; create file with read & write permission

130		DEFB	EDH		; execute open command

140		DEFB	30H

150		JR	NZ,ERROR	; jump if error

160		LD	(CHAN),DE	; save channel number of I/O

.

.

.

200	PCFN	DEFM	‘PCFILE.CMD’	; zero terminated filespec to open

210		DEFB	00H

220	CHAN	DEFW	0000H	; saved channel number

#### CLOSE DE

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	31H

Description:

Closes file on channel specified by the DE register. The Z flag is set if close successful else the Z flag is reset and the A register contains the error number.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file closed ok; reset otherwise

Example:

100		LD	DE,(CHAN)	; load DE with channel number of file to close

110		DEFB	EDH		; execute close command

120		DEFB	31H

130		JR	NZ,ERROR	; jump if error

.

.

.

200	CHAN	DEFW	0000H	; saved channel number

#### READ BC, DE, HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	32H

Description:

Read the number of bytes defined by register BC into the buffer area pointed to by register HL from the channel defined by register DE. If the Z flag is set then the read was successful, register  BC will contain the number of bytes actually read and the buffer will contain the data read. If BC is less than the value before execution of the instruction and there was no error then the end of file was reached. If an read error occurred then the Z flag is reset and BC will contain the number of bytes actually read, & register A will contain the error number.

It is possible to read 0001h to 2000h bytes in a single read.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file read ok; reset otherwise

Example:

100		LD	HL,BUFF	; load HL with pointer to buffer

110		LD	DE,(CHAN)	; load DE with channel number of file to read from

120		LD	BC,0100H	; load BC with bytes to read

130		DEFB	EDH		; execute read command

140		DEFB	32H

150		JR	NZ,ERROR	; jump if error

160		LD	A,B		; check if all bytes read

170		CP	01H

180		JR	NZ,EOF	; jump if not

190		LD	A,C

200		CP	00H

210		JR	NZ,EOF	; jump if not

.

.

.

300	BUFF	DEFS	0100H	; define a 256 byte buffer

310	CHAN	DEFW	0000H	; saved channel number

#### WRITE BC, DE, HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	33H

Description:

Write the number of bytes defined by register BC from the buffer area pointed to by register HL to the channel defined by register DE. If the Z flag is set then the write was successful, register  BC will contain the number of bytes actually read and the buffer will be unchanged. If an read error occurred then the Z flag is reset and BC will contain the number of bytes actually written & register A will contain the error number.

It is possible to write 0001h to 2000h bytes in a single write. If register BC contains 0000h then the file will be truncated at the current file pointer location.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file write ok; reset otherwise

Example:

100		LD	HL,BUFF	; load HL with pointer to buffer

110		LD	DE,(CHAN)	; load DE with channel number of file to write to

120		LD	BC,0100H	; load BC with bytes to write

130		DEFB	EDH		; execute write command

140		DEFB	33H

150		JR	NZ,ERROR	; jump if error

.

.

.

300	BUFF	DEFS	0100H	; define a 256 byte buffer

310	CHAN	DEFW	0000H	; saved channel number

#### SEEK C, DE, HL

Operation:   Emulator Instruction (DOS Int 21h Function 42h)

Format:

Opcode	Operands

DEFB	EDH

DEFB	34H -or- 3DH

Description:

Set the file position to the 32 bit signed integer value pointed to by register HL using the method defined in register C. If successful the Z flag is set and the 32 bit signed integer value pointed to by HL is updated with the new file position. If an error occurs then the Z flag is reset and register A contains the error number.

Using method 1 or 2 it is possible to position beyond the start of the file without an error, an error will occur upon a subsequent read or write to the file.

Method code:

C = 0	absolute byte offset from beginning of file (always positive double integer)
	C = 1	byte offset from current location (positive or negative double integer)
	C = 2	byte offset from end of file (positive or negative double integer)

The ED3Dh command will truncate or extend the file to the new position.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file seek ok; reset otherwise

Example:

100		LD	HL,0100H	; save new file position (00000100h)

110		LD	(BUFF),HL

120		LD	HL,0000H

130		LD	(BUFF+2),HL

140		LD	HL,(BUFF)	; load HL with pointer to buffer
(at least 4 bytes)

150		LD	DE,(CHAN)	; load DE with channel number of file to write to

160		LD	C,00H	; load BC with method code (absolute)

170		DEFB	EDH		; execute seek command

180		DEFB	34H

190		JR	NZ,ERROR	; jump if error

.

.

.

300	BUFF	DEFS	0100H	; define a 256 byte buffer

310	CHAN	DEFW	0000H	; saved channel number

#### ERROR A, BC, HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	35H

Description:

Builds error message in buffer pointed to by register HL for the error number in register A. If the length of the buffer is not able to hold the entire message the message is truncated to fit and register A contains range error, else register A is set to zero.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	always set

Example:

100		LD	DE,(CHAN)	; load DE with channel number of file to close

110		DEFB	EDH		; execute close command

120		DEFB	31H

130		JR	NZ,ERROR	; jump if error, error number is in register A

.

.

.

200	ERROR	LD	HL,ERRMSG	; load HL with pointer to message buffer

210		DEFB	EDH		; execute error message command

220		DEFB	35H		; upon return the error message is in

.			; buffer, zero terminated

.

.

300	ERRMSG	DEFS	0100H	; define a 256 byte buffer

310	CHAN	DEFW	0000H	; saved channel number

#### FINDFIRST BC, DE, HL

Operation:   Emulator Instruction (DOS Int 21h Function 4Eh)

Format:

Opcode	Operands

DEFB	EDH

DEFB	36H

Description:

Given a file specification in the form of an ASCIIZ string, searches the default or specified directory on the default or specified disk drive for the first matching file. If a match is found the buffer pointed to by register HL will be loaded with the file data found and the Z flag is set, else the Z flag is reset and register A contains the error number.

Wildcard characters ? and * are allowed in the filename. Files found are based on the attributes set in the DE register.

WARNING: Buffer must be at least 43 bytes long but not longer than 255 bytes.

File Attributes:

bit 0 of DE	read-only
	bit 1 of DE	hidden
	bit 2 of DE	system
	bit 3 of DE	volume labels (if set only volume labels are returned)
	bit 4 of DE	subdirectory; entry excluded from normal searches
	bit 5 of DE	archive

Buffer Structure:

bytes 0-20	reserved for DOS on subsequent calls
	byte 21	attribute of matching file
	bytes 22-23	file time
	bytes 24-25	file date
	bytes 26-29	file size (32 bit integer)
	bytes 30-42	filename & extension in form of ASCIIZ string

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file match found; reset otherwise

Example:

100		LD	HL,BUFF	; load HL with address of filespec

110		LD	BC,43	; buffer must be 43 bytes long

120		LD	DE,01H	; find all normal and read only files

130		DEFB	EDH		; execute findfirst command

140		DEFB	36H

150		JR	NZ,ERROR	; jump if no matching files

.			; buffer filled with directory entry,

.			; filename starts at byte 30

.

200	BUFF	DEFM	‘*.*’	; zero terminated filespec to search for

210		DEFB	00H

220		DEFS	39		; buffer must be 43 bytes long

#### FINDNEXT BC, HL

Operation:   Emulator Instruction (DOS Int 21h Function 4Fh)

Format:

Opcode	Operands

DEFB	EDH

DEFB	37H

Description:

Assuming a successful previous FINDFIRST function the FINDNEXT function finds the next file in the default or specified directory on the default or specified disk drive that matches the original file specification. If a match is found the buffer pointed to by register HL will be loaded with the file data found and the Z flag is set, else the Z flag is reset and register A contains the error number.

WARNING: Buffer must be at least 43 bytes long but not longer than 255 bytes.

Buffer Structure:

bytes 0-20	reserved for DOS on subsequent calls
	byte 21	attribute of matching file
	bytes 22-23	file time
	bytes 24-25	file date
	bytes 26-29	file size (32 bit integer)
	bytes 30-42	filename & extension in form of ASCIIZ string

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file match found; reset otherwise

Example:

100		LD	HL,BUFF	; load HL with address of filespec

110		LD	BC,43	; buffer must be 43 bytes long

120		DEFB	EDH		; execute findnext command

130		DEFB	37H

140		JR	NZ,ERROR	; jump if no more matching files

.			; buffer filled with directory entry,

.			; filename starts at byte 30

.

200	BUFF	DEFS	43		; buffer must be 43 bytes long

#### LOAD BC, HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	39H

Description:

Open and load the object file specified by the path pointed to by HL. The path must conform to PC standards and be zero terminated. Register BC contains a load offset that is added to the object file's load addresses. Normally BC is zero.

If the load is successful the Z flag is set and if the object file contained an execution address then the C flag is also set and register BC contains the execution address. If the file was not found or was not a valid object file then the Z flag is reset and the A register contains the error number.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file opened ok; reset otherwise

Example:

100		LD	HL,PCFN	; load HL with address of filespec

110		LD	BC,00H	; load BC w/zero (no load offset)

120		DEFB	EDH		; execute load command

130		DEFB	39H

140		JR	NZ,ERROR	; jump if error

150		JR	NC,EXIT	; exit if no exec address found

160		DI			; disable interrupts

170		PUSH	BC		; push exec address on stack

180		RET			; execute loaded program

.

.

.

200	PCFN	DEFM	‘GALAXY.CMD’	; zero terminated filespec to load

210		DEFB	00H

#### CHDIR HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	3AH

Description:

Changes the current drive and directory to the path pointed to by the register HL. If the command is successful the Z flag is set else the Z flag is reset and the A register contains the error number.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file chdir ok; reset otherwise

Example:

100		LD	HL,BUFF	; load HL with address of path

110		DEFB	EDH		; execute chdir command

120		DEFB	3AH

130		JR	NZ,ERROR	; jump if error

.

.

.

200	BUFF	DEFM	‘C:\TRS80’	; zero terminated path to change to

210		DEFB	00H

#### GETDIR BC, HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	3BH

Description:

Gets the current working directory and drive. The path and drive are stored in the buffer pointed to by the register HL. If the command is successful the Z flag is set else the Z flag is reset and the A register contains the error number. If the full path is longer than the buffer an error will occur.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file getdir ok; reset otherwise

Example:

100		LD	HL,BUFF	; load HL with address of path

110		LD	BC,128	; save length of buffer

120		DEFB	EDH		; execute getdir command

130		DEFB	3BH

140		JR	NZ,ERROR	; jump if error

.			; upon return the path is in buffer,

.			; zero terminated

.

200	BUFF	DEFS	128		; create a buffer long enough to hold path

#### PATHOPEN BC, DE, HL

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	3EH

Description:

Opens the file specified by the path pointed to by HL. The path must conform to PC standards and be zero terminated. The BC register is used to define the type of file access. If the file is being created the DE register is used to define the access permission.

If the open is successful the Z flag is set and the file’s channel is returned in the DE register else the Z flag is reset, DE is loaded with FFFFh and the A register contains the error number.

Unlike the OPEN command this command is used in writing floppy & hard disk drivers for TRS-80 DOS's.

If no path is given the current floppy path will be used (set in the floppy selection screen). If no extension is given the extension .DSK will be used.

If 'HARDX-YY' (where X= '1', '3' or '4' and YY= a value of 0-7) is given as the filename then one of the 8 hard disk filenames (16 in the Model 3/4 emulator) defined in the configuration screen will be opened.

File Access:

bits 0-1 of BC:
	00	open file read only
	01	open file write only
	10	open file read/write

bit 6 of BC	create file if doesn't exist (create)
	bit 7 of BC	used only with bit 6 set. If file exists, an error is returned
	bit 9 of BC	if file exists then file is truncated (truncate)
	bit 10 of BC	the file pointer will be set to the end of the file prior to each write (append)

File Access Permission:

bit 7 of DE	if set, permission to write to file
	bit 8 of DE	if set, permission to read to file

NOTE:	Only has effect when file is being created. To allow read & write access both bits must be set.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file opened ok; reset otherwise

Example:

100		LD	HL,HARD	; load HL with address of filespec

110		LD	BC,0002H	; open in read/write mode

120		DEFB	EDH		; execute open command

130		DEFB	3EH

140		JR	NZ,ERROR	; jump if error

150		LD	(CHAN),DE	; save channel number of I/O

.

.

.

200	HARD	DEFM	‘HARD4-0’	; open defined Model 4 mode hard drive #0

210		DEFB	00H

220	CHAN	DEFW	0000H	; saved channel number

If in the configuration screen the Model 4 drive #0 is defined as:

C:\TRS-80\LDOS0.HDV

Then, if the file exists, it will be opened for read/write I/O and the channel number returned in DE.

#### CLOSEALL DE

Operation:   Emulator Instruction

Format:

Opcode	Operands

DEFB	EDH

DEFB	3FH

Description:

Closes file on channel specified by the DE register or closes all open files if DE=FFFFh. The Z flag is set if close successful else the Z flag is reset and the A register contains the error number.

If DE=FFFFh then no error is reported even if no files are closed.

M CYCLES: 4	T STATES: 4	4 MHZ E.T.: 1.00

Condition Bits Affected:

Z:	Set if file closed ok; reset otherwise

Example:

100		LD	DE,FFFFH	; load DE FFFFh to close all files

110		DEFB	EDH		; execute close command

120		DEFB	3FH

130		JR	NZ,ERROR	; jump if error

.

.

.